Remote Preset Editing via MIDI SysEx

Draft #30 EOS 4.00

17 February, 1999

Brian Clark

SPECIFICATION:

Focus:        What's it all about.

General Info: Device Inquiry messages

Dumps:        Sample Dump Standard like protocol

Format:       The technical specifics.

Parameters:   A List of ID's and their meanings.

Commands:     A List of Commands and Examples.

Focus:

The main focus is to provide an easy to use, easy to implement, elegant

protocol for the specific function of editing E4 presets from a remote

system.  Typically this would be a computer, with the potential for a

large, colorful, graphical interface, superior to the standard E4 front

panel display.

The protocol consists of a data format usable over MIDI, as well as

any other transfer media.  The data format is simple, flexible, and easy

to expand while maintaining backwards compatibility.

There are a few basic necessary functions that make up a Remote

Preset Editor/Librarian package.

- the ability to select the Preset to edit.

- the ability to select the Voice or Group of Voices to edit.

- the ability to send a new individual parameter value to the E4.

- the ability to request from the E4 the value of an individual parameter.

- the ability to send a Dump of parameters to the E4.

- the ability to request from the E4 a Dump of parameters.

- the ability to Name a Preset.

- the ability to request the Name of a Preset/Sample.

- the ability to Catalogue the Folders/Banks/Presets/Samples of a disk.

General Information:

The E4 line of products support the Non-Real Time System Exclusive

General Information Device Inquiry messages (sub-ID#1 = 06).

Format:  {F0h,7Eh,<device ID>,06h,01h,F7}

{F0h,7Eh,<device ID>} = Universal System Exclusive Non-real time header.

06h  = General Information (sub-ID#1)

01h  = Identity Request (sub-ID#2)

F7h  = EOX

Response: {F0h,7Eh,<device ID>,06h,02h,mm,ff,ff,dd,dd,ss,ss,ss,ss,F7}

{F0h,7Eh,<device ID>} = Universal System Exclusive Non-real time header.

<device ID>  = ID the hunk of metal and plastic is set to.

0 - 126 are Unique ID's, 127 is an All Broadcast ID.

06h  = General Information (sub-ID#1)

02h  = Identity Reply (sub-ID#2)

mm  = 18h (Manufacturers System Exclusive id code)

ff ff  = 01h,04h (Device family code, 14 bits, LSB first)

dd dd  = Device family member code, 14 bits, LSB first

00h,05h = E4

01h,05h = E64

02h,05h = E4k

03h,05h = E64FX

04h,05h = E4XT

05h,05h = E4X

06h,05h = E6400

07h,05h = E4XT ULTRA

08h,05h = E6400 ULTRA

ss ss ss ss  = Software revision level, 4 ASCII characters.

example:  4  .  0  0  = version 4.00

F7h  = EOX

E4 Specific Data Format:

The transfer of Data is organized in the following ways.

Format:  <Header>,<Command>,<Count*>,<Packet1>,<Packet2>,...,<Checksum*>,<EOX>.

<Header>

F0h = sysex message

18h = EMU ID

21h = E4 ID

xxh = Device ID

00h - 7Eh are Unique

7Fh is an All Broadcast

55h = Special Editor designator byte

<Command> determines what function to perform, such as:

Parameter Edit/Request

Preset/Sample Name/Request

Dump/Request

<Count> Gives the number of bytes in a message, such as

Parameter Edits of multiple parameters.  *Only used

in variable sized messages.

- Packets take several forms, depending on the command.  Some commands may

not require a packet at all.

<Packet> = <2 Byte ID>,<2 Byte Data>.

The main packet has 2 parts;  An ID portion and the Data

portion.  This is used for editing a parameter.  The ID

chooses the parameter, and the Data is the new value.

Every Parameter has a unique ID.  This way, parameters can be

individually manipulated.  However, these Parameter ID's operate

on a specific Voice/Group/Sample, as well as Global parameters.

The ID Byte is the Parameter ID, a 14-bit unsigned number split

into 2 MIDI bytes, and sent LSB first.

The Data is a 14 bit signed or unsigned number split into 2

7-bit MIDI bytes, and sent LSB first.

<Packet> = <2 Byte ID>

The request packet requires only the Parameter ID, or Preset/

Sample number.  Typically used in a request for data from a

particular Parameter.

<Packet> = <Preset/Sample #>,<ASCII string>

The Name packet contains the Preset or Sample number, followed

by the name as a string of ASCII characters.

<Packet> = <Preset #>

Selecting another Preset.

<Checksum> = 1 Byte XOR(1's compliment) of the sum of the DATA bytes in the message.

*Only used in certain messages.  A checksum value of 7Fh means "ignore

checksum".

<EOX> = F7h end of SYSEX.

The following is a list of editable parameters and their ID's:

- Values recieved that are outside the min/max range will be clipped

to within that range.

- Preset and Sample Number Ranges may vary if Preset Flash, Sample ROM or

Sample Flash are present.  Preset and Sample information for ROM/Flash

locations may be requested, but cannot be changed.

GLOBAL:

(LSB,MSB)

E4_PRESET_TRANSPOSE,       id = 0 (00h,00h)      min = -24;  max = +24  (semitones)

E4_PRESET_VOLUME,          id = 1 (01h,00h)      min = -96;  max = +10  (dB)

E4_PRESET_CTRL_A,          id = 2 (02h,00h)      min = -1;   max = 127  (-1 = off)

E4_PRESET_CTRL_B,          id = 3 (03h,00h)      min = -1;   max = 127  (-1 = off)

E4_PRESET_CTRL_C,          id = 4 (04h,00h)      min = -1;   max = 127  (-1 = off)

E4_PRESET_CTRL_D,          id = 5 (05h,00h)      min = -1;   max = 127  (-1 = off)

E4_PRESET_FX_A_ALGORITHM,  id = 6 (06h,00h)      min = 0;  max =  44 (see below)

E4_PRESET_FX_A_PARM_0,     id = 7 (07h,00h)      min = 0;  max =  90

E4_PRESET_FX_A_PARM_1,     id = 8 (08h,00h)      min = 0;  max = 127

E4_PRESET_FX_A_PARM_2,     id = 9 (09h,00h)      min = 0;  max = 127

E4_PRESET_FX_A_AMT_0,      id = 10 (0Ah,00h)     min = 0;  max = 100

E4_PRESET_FX_A_AMT_1,      id = 11 (0Bh,00h)     min = 0;  max = 100

E4_PRESET_FX_A_AMT_2,      id = 12 (0Ch,00h)     min = 0;  max = 100

E4_PRESET_FX_A_AMT_3,      id = 13 (0Dh,00h)     min = 0;  max = 100

E4_PRESET_FX_B_ALGORITHM,  id = 14 (0Eh,00h)     min = 0;  max =  27 (see below)

E4_PRESET_FX_B_PARM_0,     id = 15 (0Fh,00h)     min = 0;  max = 127

E4_PRESET_FX_B_PARM_1,     id = 16 (10h,00h)     min = 0;  max = 127

E4_PRESET_FX_B_PARM_2,     id = 17 (11h,00h)     min = 0;  max = 127

E4_PRESET_FX_B_AMT_0,      id = 18 (12h,00h)     min = 0;  max = 100

E4_PRESET_FX_B_AMT_1,      id = 19 (13h,00h)     min = 0;  max = 100

E4_PRESET_FX_B_AMT_2,      id = 20 (14h,00h)     min = 0;  max = 100

E4_PRESET_FX_B_AMT_3,      id = 21 (15h,00h)     min = 0;  max = 100

*** id 22 not used ***

LINKS:

E4_LINK_PRESET,            id = 23 (17h,00h)     min =   0;  max = 999(1255)

E4_LINK_VOLUME,            id = 24 (18h,00h)     min = -96;  max = +10

E4_LINK_PAN,               id = 25 (19h,00h)     min = -64;  max = +63

E4_LINK_TRANSPOSE,         id = 26 (1Ah,00h)     min = -24;  max = +24

E4_LINK_FINE_TUNE,         id = 27 (1Bh,00h)     min = -64;  max = +64

E4_LINK_KEY_LOW,           id = 28 (1Ch,00h)     min = 0;  max = 127  (C-2 -> G8)

E4_LINK_KEY_LOWFADE,       id = 29 (1Dh,00h)     min = 0;  max = 127

E4_LINK_KEY_HIGH,          id = 30 (1Eh,00h)     min = 0;  max = 127  (C-2 -> G8)

E4_LINK_KEY_HIGHFADE,      id = 31 (1Fh,00h)     min = 0;  max = 127

E4_LINK_VEL_LOW,           id = 32 (20h,00h)     min = 0;  max = 127

E4_LINK_VEL_LOWFADE,       id = 33 (21h,00h)     min = 0;  max = 127

E4_LINK_VEL_HIGH,          id = 34 (22h,00h)     min = 0;  max = 127

E4_LINK_VEL_HIGHFADE,      id = 35 (23h,00h)     min = 0;  max = 127

*** id 36 not used ***

VOICES:

E4_GEN_GROUP_NUM,          id = 37 (25h,00h)     min =   1;  max =  32

E4_GEN_SAMPLE,             id = 38 (26h,00h)     min =   0;  max = 999(2999)

E4_GEN_VOLUME,             id = 39 (27h,00h)     min = -96;  max = +10

E4_GEN_PAN,                id = 40 (28h,00h)     min = -64;  max = +63

E4_GEN_CTUNE,              id = 41 (29h,00h)     min = -72;  max = +24  (Voice only)

E4_GEN_FTUNE,              id = 42 (2Ah,00h)     min = -64;  max = +64

E4_GEN_XPOSE,              id = 43 (2Bh,00h)     min = -24;  max = +24  (Voice only)

E4_GEN_ORIG_KEY,           id = 44 (2Ch,00h)     min =   0;  max = 127  (60 = C3, Sample only)

E4_GEN_KEY_LOW,            id = 45 (2Dh,00h)     min = 0;  max = 127  (C-2 -> G8)

E4_GEN_KEY_LOWFADE,        id = 46 (2Eh,00h)     min = 0;  max = 127

E4_GEN_KEY_HIGH,           id = 47 (2Fh,00h)     min = 0;  max = 127  (C-2 -> G8)

E4_GEN_KEY_HIGHFADE,       id = 48 (30h,00h)     min = 0;  max = 127

E4_GEN_VEL_LOW,            id = 49 (31h,00h)     min = 0;  max = 127

E4_GEN_VEL_LOWFADE,        id = 50 (32h,00h)     min = 0;  max = 127

E4_GEN_VEL_HIGH,           id = 51 (33h,00h)     min = 0;  max = 127

E4_GEN_VEL_HIGHFADE,       id = 52 (34h,00h)     min = 0;  max = 127

E4_GEN_RT_LOW,             id = 53 (35h,00h)     min = 0;  max = 127  (Voice only)

E4_GEN_RT_LOWFADE,         id = 54 (36h,00h)     min = 0;  max = 127  (Voice only)

E4_GEN_RT_HIGH,            id = 55 (37h,00h)     min = 0;  max = 127  (Voice only)

E4_GEN_RT_HIGHFADE,        id = 56 (38h,00h)     min = 0;  max = 127  (Voice only)

Tuning:

E4_VOICE_NON_TRANSPOSE,    id = 57 (39h,00h)     min = 0;  max = 1

0 = off

1 = on

E4_VOICE_CHORUS_AMOUNT,    id = 58 (3Ah,00h)     min =    0;  max = 100 (%)

E4_VOICE_CHORUS_WIDTH,     id = 59 (3Bh,00h)     min = -128;  max =   0

Displayed Value:

int pct = ((val + 128) * 100) / 128;

sprintf(buf,"%3d%%", pct);

E4_VOICE_CHORUS_X,         id = 60 (3Ch,00h)     min =  -32;  max = +32 (ms)

Chorus initial ITD(Inter-Aural Time Delay):

Adjusts the delay of the left and right sounds.

Positive numbers delay the left channel more.

Negative numbers delay the right channel more.

(+ or -)   (+ or -)

0.000ms  =  0

0.045ms  =  1

0.090ms  =  2

0.136ms  =  3

0.181ms  =  4

0.226ms  =  5

0.272ms  =  6

0.317ms  =  7

0.362ms  =  8

0.408ms  =  9

0.453ms  =  10

0.498ms  =  11

0.544ms  =  12

0.589ms  =  13

0.634ms  =  14

0.680ms  =  15

0.725ms  =  16

0.770ms  =  17

0.816ms  =  18

0.861ms  =  19

0.907ms  =  20

0.952ms  =  21

0.997ms  =  22

1.043ms  =  23

1.088ms  =  24

1.133ms  =  25

1.179ms  =  26

1.224ms  =  27

1.269ms  =  28

1.315ms  =  29

1.360ms  =  30

1.405ms  =  31

1.451ms  =  32

E4_VOICE_DELAY,            id = 61 (3Dh,00h)     min = 0;  max = 10000  (ms)

E4_VOICE_START_OFFSET,     id = 62 (3Eh,00h)     min = 0;  max =   127

E4_VOICE_GLIDE_RATE,       id = 63 (3Fh,00h)     min = 0;  max = 127 (sec/oct)

(Portamento)

Here are the function and tables for the displayed values:

This is for E4_VOICE_GLIDE_RATE only.

LOCAL Void cnv_glide_rate(Long val, Char *buf)

{

int msec = (envunits1[val] * 1000 + envunits2[val] * 10) / 5;

sprintf(buf, "%2d.%03dsec/oct", msec / 1000, msec % 1000);

}

const unsigned char            envunits1[] =

{

0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0,

1, 1, 1, 1, 1, 1, 1, 1,

1, 1, 1, 1, 2, 2, 2, 2,

2, 2, 2, 3, 3, 3, 3, 3,

4, 4, 4, 4, 5, 5, 5, 5,

6, 6, 7, 7, 7, 8, 8, 9,

9, 10, 11, 11, 12, 13, 13, 14,

15, 16, 17, 18, 19, 20, 22, 23,

24, 26, 28, 30, 32, 34, 36, 38,

41, 44, 47, 51, 55, 59, 64, 70,

76, 83, 91, 100, 112, 125, 142, 163,

};

const unsigned char            envunits2[] =

{

00, 01, 02, 03, 04, 05, 06, 07,

8, 9, 10, 11, 12, 13, 14, 15,

16, 17, 18, 19, 20, 21, 22, 23,

25, 26, 28, 29, 32, 34, 36, 38,

41, 43, 46, 49, 52, 55, 58, 62,

65, 70, 74, 79, 83, 88, 93, 98,

04, 10, 17, 24, 31, 39, 47, 56,

65, 74, 84, 95, 06, 18, 31, 44,

59, 73, 89, 06, 23, 42, 62, 82,

04, 28, 52, 78, 05, 34, 64, 97,

32, 67, 06, 46, 90, 35, 83, 34,

87, 45, 06, 70, 38, 11, 88, 70,

56, 49, 48, 53, 65, 85, 13, 50,

97, 54, 24, 06, 02, 15, 44, 93,

64, 60, 84, 41, 34, 70, 56, 03,

22, 28, 40, 87, 9, 65, 36, 69,

};

0.000sec/oct to 0.046sec/oct by 2's

0.050

0.052

0.056

0.058

0.064

0.068

0.072

0.076

0.082

0.086

0.092

0.098

0.104

0.110

0.116

0.124

0.130

0.140

0.148

0.158

0.166

0.176

0.186

0.196

0.208

0.220

0.234

0.248

0.262

0.278

0.294

0.312

0.330

0.348

0.368

0.390

0.412

0.436

0.462

0.488

0.518

0.546

0.578

0.612

0.646

0.684

0.724

0.764

0.808

0.856

0.904

0.956

1.010

1.068

1.128

1.194

1.264

1.334

1.412

1.492

1.580

1.670

1.766

1.868

1.974

2.090

2.212

2.340

2.476

2.622

2.776

2.940

3.112

3.298

3.496

3.706

3.930

4.170

4.426

4.700

4.994

5.308

5.648

6.012

6.404

6.830

7.288

7.786

8.328

8.920

9.568

10.282

11.068

11.940

12.912

14.006

15.244

16.656

18.280

20.174

22.148

25.130

28.472

32.738

E4_VOICE_GLIDE_CURVE,      id = 64 (40h,00h)     min = 0;  max = 8

linear -> exponential;  0 = linear, 8 = most exponential curve

E4_VOICE_SOLO,             id = 65 (41h,00h)     min = 0;  max = 8

"Monophonic" mode.

0 = Off

1 = Multiple Trigger

2 = Melody (last)

3 = Melody (low)

4 = Melody (high)

5 = Synth (last)

6 = Synth (low)

7 = Synth (high)

8 = Fingered Glide

E4_VOICE_ASSIGN_GROUP,     id = 66 (42h,00h)     min = 0;  max = 23

0   =  Poly All

1   =  Poly16 A

2   =  Poly16 B

3   =  Poly 8 A

4   =  Poly 8 B

5   =  Poly 8 C

6   =  Poly 8 D

7   =  Poly 4 A

8   =  Poly 4 B

9   =  Poly 4 C

10  =  Poly 4 D

11  =  Poly 2 A

12  =  Poly 2 B

13  =  Poly 2 C

14  =  Poly 2 D

15  =  Mono A

16  =  Mono B

17  =  Mono C

18  =  Mono D

19  =  Mono E

20  =  Mono F

21  =  Mono G

22  =  Mono H

23  =  Mono I

E4_VOICE_LATCHMODE,        id = 67 (43h,00h)     min =    0;  max =   1

0 = off

1 = on

Amplifier:

E4_VOICE_VOLENV_DEPTH,     id = 68 (44h,00h)     min = 0;  max = 16

Amp Env Depth:    -96dB to -48dB by 3's

E4_VOICE_SUBMIX,           id = 69 (45h,00h)     min = -1;  max =   3

Submix: -1 = voice

0 = main

1 = sub1

2 = sub2

3 = sub3

(if has Octopus card)

4 = sub4

5 = sub5

6 = sub6

7 = sub7

E4_VOICE_VENV_SEG0_RATE,   id = 70 (46h,00h)     min = 0;  max = 127 (Atk1 Rate)

E4_VOICE_VENV_SEG0_TGTLVL, id = 71 (47h,00h)     min = 0;  max = 100 (Atk1 Level%)

E4_VOICE_VENV_SEG1_RATE,   id = 72 (48h,00h)     min = 0;  max = 127 (Dcy1 Rate)

E4_VOICE_VENV_SEG1_TGTLVL, id = 73 (49h,00h)     min = 0;  max = 100 (Dcy1 Level%)

E4_VOICE_VENV_SEG2_RATE,   id = 74 (4Ah,00h)     min = 0;  max = 127 (Rls1 Rate)

E4_VOICE_VENV_SEG2_TGTLVL, id = 75 (4Bh,00h)     min = 0;  max = 100 (Rls1 Level%)

E4_VOICE_VENV_SEG3_RATE,   id = 76 (4Ch,00h)     min = 0;  max = 127 (Atk2 Rate)

E4_VOICE_VENV_SEG3_TGTLVL, id = 77 (4Dh,00h)     min = 0;  max = 100 (Atk2 Level%)

E4_VOICE_VENV_SEG4_RATE,   id = 78 (4Eh,00h)     min = 0;  max = 127 (Dcy2 Rate)

E4_VOICE_VENV_SEG4_TGTLVL, id = 79 (4Fh,00h)     min = 0;  max = 100 (Dcy2 Level%)

E4_VOICE_VENV_SEG5_RATE,   id = 80 (50h,00h)     min = 0;  max = 127 (Rls2 Rate)

E4_VOICE_VENV_SEG5_TGTLVL, id = 81 (51h,00h)     min = 0;  max = 100 (Rls2 Level%)

/*

* Filter Parameters:   See Filter section below for filter types.

*/

E4_VOICE_FTYPE,            id = 82 (52h,00h)     min = 0;  max = variable

E4_VOICE_FMORPH,           id = 83 (53h,00h)     min = 0;  max = 255

E4_VOICE_FKEY_XFORM,       id = 84 (54h,00h)     min = 0;  max = 127

E4_VOICE_FILT_GEN_PARM1,   id = 85 (55h,00h)     min = 0;  max = 255 (reserved for future expansion)

E4_VOICE_FILT_GEN_PARM2,   id = 86 (56h,00h)     min = 0;  max = 255 (reserved for future expansion)

E4_VOICE_FILT_GEN_PARM3,   id = 87 (57h,00h)     min = 0;  max = 255

E4_VOICE_FILT_GEN_PARM4,   id = 88 (58h,00h)     min = 0;  max = 255

E4_VOICE_FILT_GEN_PARM5,   id = 89 (59h,00h)     min = 0;  max = 255

E4_VOICE_FILT_GEN_PARM6,   id = 90 (5Ah,00h)     min = 0;  max = 255

E4_VOICE_FILT_GEN_PARM7,   id = 91 (5Bh,00h)     min = 0;  max = 255

E4_VOICE_FILT_GEN_PARM8,   id = 92 (5Ch,00h)     min = 0;  max = 255

E4_VOICE_FENV_SEG0_RATE,   id = 93 (5Dh,00h)     min = 0;  max = 127 (Atk1 Rate)

E4_VOICE_FENV_SEG0_TGTLVL, id = 94 (5Eh,00h)     min = 0;  max = 100 (Atk1 Level%)

E4_VOICE_FENV_SEG1_RATE,   id = 95 (5Fh,00h)     min = 0;  max = 127 (Dcy1 Rate)

E4_VOICE_FENV_SEG1_TGTLVL, id = 96 (60h,00h)     min = 0;  max = 100 (Dcy1 Level%)

E4_VOICE_FENV_SEG2_RATE,   id = 97 (61h,00h)     min = 0;  max = 127 (Rls1 Rate)

E4_VOICE_FENV_SEG2_TGTLVL, id = 98 (62h,00h)     min = 0;  max = 100 (Rls1 Level%)

E4_VOICE_FENV_SEG3_RATE,   id = 99 (63h,00h)     min = 0;  max = 127 (Atk2 Rate)

E4_VOICE_FENV_SEG3_TGTLVL, id = 100 (64h,00h)    min = 0;  max = 100 (Atk2 Level%)

E4_VOICE_FENV_SEG4_RATE,   id = 101 (65h,00h)    min = 0;  max = 127 (Dcy2 Rate)

E4_VOICE_FENV_SEG4_TGTLVL, id = 102 (66h,00h)    min = 0;  max = 100 (Dcy2 Level%)

E4_VOICE_FENV_SEG5_RATE,   id = 103 (67h,00h)    min = 0;  max = 127 (Rls2 Rate)

E4_VOICE_FENV_SEG5_TGTLVL, id = 104 (68h,00h)    min = 0;  max = 100 (Rls2 Level%)

E4_VOICE_LFO_RATE,         id = 105 (69h,00h)    min = 0;  max = 127

Conversions for displayed values:

Void cnv_lfo_rate(Long val, Char *buf)

{

sprintf (buf, "%2d.%02d", lfounits1[val], lfounits2[val]);

}

const unsigned char            lfounits1[] =

{

0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0,

1, 1, 1, 1, 1, 1, 1, 1,

1, 1, 1, 1, 1, 1, 1, 1,

2, 2, 2, 2, 2, 2, 2, 2,

2, 2, 2, 2, 2, 3, 3, 3,

3, 3, 3, 3, 3, 3, 3, 4,

4, 4, 4, 4, 4, 4, 4, 4,

5, 5, 5, 5, 5, 5, 5, 6,

6, 6, 6, 6, 6, 7, 7, 7,

7, 7, 7, 8, 8, 8, 8, 8,

9, 9, 9, 9, 10, 10, 10, 10,

10, 11, 11, 11, 11, 12, 12, 12,

13, 13, 13, 13, 14, 14, 14, 15,

15, 15, 16, 16, 17, 17, 17, 18,

};

const unsigned char            lfounits2[] =

{

8, 11, 15, 18, 21, 25, 28, 32,

35, 39, 42, 46, 50, 54, 58, 63,

67, 71, 76, 80, 85, 90, 94, 99,

04, 10, 15, 20, 25, 31, 37, 42,

48, 54, 60, 67, 73, 79, 86, 93,

00, 07, 14, 21, 29, 36, 44, 52,

60, 68, 77, 85, 94, 03, 12, 21,

31, 40, 50, 60, 70, 81, 91, 02,

13, 25, 36, 48, 60, 72, 84, 97,

10, 23, 37, 51, 65, 79, 94, 8,

24, 39, 55, 71, 88, 04, 21, 39,

57, 75, 93, 12, 32, 51, 71, 92,

13, 34, 56, 78, 00, 23, 47, 71,

95, 20, 46, 71, 98, 25, 52, 80,

9, 38, 68, 99, 30, 61, 93, 26,

60, 94, 29, 65, 01, 38, 76, 14,

};

E4_VOICE_LFO_SHAPE,        id = 106 (6Ah,00h)    min = 0;  max =   7

Shape: 0 = triangle

1 = sine

2 = sawtooth

3 = square

4 = 0,1,0,-1

5 = C,E,G,C

6 = C,D,F,G

7 = 8st Pent

E4_VOICE_LFO_DELAY,        id = 107 (6Bh,00h)    min = 0;  max = 127

E4_VOICE_LFO_VAR,          id = 108 (6Ch,00h)    min = 0;  max = 100 (%)

E4_VOICE_LFO_SYNC,         id = 109 (6Dh,00h)    min = 0;  max =   1

Sync:  0 = key sync

1 = free run

E4_VOICE_LFO2_RATE,        id = 110 (6Eh,00h)    min = 0;  max = 127 (as above)

E4_VOICE_LFO2_SHAPE,       id = 111 (6Fh,00h)    min = 0;  max =   7 (as above)

E4_VOICE_LFO2_DELAY,       id = 112 (70h,00h)    min = 0;  max = 127

E4_VOICE_LFO2_VAR,         id = 113 (71h,00h)    min = 0;  max = 100 (%)

E4_VOICE_LFO2_SYNC,        id = 114 (72h,00h)    min = 0;  max =   1 (as above)

E4_VOICE_LFO2_OP0_PARM,    id = 115 (73h,00h)    min = 0;  max = 10  (Lag 0: )

E4_VOICE_LFO2_OP1_PARM,    id = 116 (74h,00h)    min = 0;  max = 10  (Lag 1: )

E4_VOICE_AENV_SEG0_RATE,   id = 117 (75h,00h)    min = 0;  max = 127 (Atk1 Rate)

E4_VOICE_AENV_SEG0_TGTLVL, id = 118 (76h,00h)    min = 0;  max = 100 (Atk1 Level%)

E4_VOICE_AENV_SEG1_RATE,   id = 119 (77h,00h)    min = 0;  max = 127 (Dcy1 Rate)

E4_VOICE_AENV_SEG1_TGTLVL, id = 120 (78h,00h)    min = 0;  max = 100 (Dcy1 Level%)

E4_VOICE_AENV_SEG2_RATE,   id = 121 (79h,00h)    min = 0;  max = 127 (Rls1 Rate)

E4_VOICE_AENV_SEG2_TGTLVL, id = 122 (7Ah,00h)    min = 0;  max = 100 (Rls1 Level%)

E4_VOICE_AENV_SEG3_RATE,   id = 123 (7Bh,00h)    min = 0;  max = 127 (Atk2 Rate)

E4_VOICE_AENV_SEG3_TGTLVL, id = 124 (7Ch,00h)    min = 0;  max = 100 (Atk2 Level%)

E4_VOICE_AENV_SEG4_RATE,   id = 125 (7Dh,00h)    min = 0;  max = 127 (Dcy2 Rate)

E4_VOICE_AENV_SEG4_TGTLVL, id = 126 (7Eh,00h)    min = 0;  max = 100 (Dcy2 Level%)

E4_VOICE_AENV_SEG5_RATE,   id = 127 (7Fh,00h)    min = 0;  max = 127 (Rls2 Rate)

E4_VOICE_AENV_SEG5_TGTLVL, id = 128 (00h,01h)    min = 0;  max = 100 (Rls2 Level%)

Cords:

E4_VOICE_CORD0_SRC,        id = 129 (01h,01h)    min =    0;  max =  255

E4_VOICE_CORD0_DST,        id = 130 (02h,01h)    min =    0;  max =  255

E4_VOICE_CORD0_AMT,        id = 131 (03h,01h)    min = -100;  max = +100

E4_VOICE_CORD1_SRC,        id = 132 (04h,01h)    min =    0;  max =  255

E4_VOICE_CORD1_DST,        id = 133 (05h,01h)    min =    0;  max =  255

E4_VOICE_CORD1_AMT,        id = 134 (06h,01h)    min = -100;  max = +100

E4_VOICE_CORD2_SRC,        id = 135 (07h,01h)    min =    0;  max =  255

E4_VOICE_CORD2_DST,        id = 136 (08h,01h)    min =    0;  max =  255

E4_VOICE_CORD2_AMT,        id = 137 (09h,01h)    min = -100;  max = +100

E4_VOICE_CORD3_SRC,        id = 138 (0Ah,01h)    min =    0;  max =  255

E4_VOICE_CORD3_DST,        id = 139 (0Bh,01h)    min =    0;  max =  255

E4_VOICE_CORD3_AMT,        id = 140 (0Ch,01h)    min = -100;  max = +100

E4_VOICE_CORD4_SRC,        id = 141 (0Dh,01h)    min =    0;  max =  255

E4_VOICE_CORD4_DST,        id = 142 (0Eh,01h)    min =    0;  max =  255

E4_VOICE_CORD4_AMT,        id = 143 (0Fh,01h)    min = -100;  max = +100

E4_VOICE_CORD5_SRC,        id = 144 (10h,01h)    min =    0;  max =  255

E4_VOICE_CORD5_DST,        id = 145 (11h,01h)    min =    0;  max =  255

E4_VOICE_CORD5_AMT,        id = 146 (12h,01h)    min = -100;  max = +100

E4_VOICE_CORD6_SRC,        id = 147 (13h,01h)    min =    0;  max =  255

E4_VOICE_CORD6_DST,        id = 148 (14h,01h)    min =    0;  max =  255

E4_VOICE_CORD6_AMT,        id = 149 (15h,01h)    min = -100;  max = +100

E4_VOICE_CORD7_SRC,        id = 150 (16h,01h)    min =    0;  max =  255

E4_VOICE_CORD7_DST,        id = 151 (17h,01h)    min =    0;  max =  255

E4_VOICE_CORD7_AMT,        id = 152 (18h,01h)    min = -100;  max = +100

E4_VOICE_CORD8_SRC,        id = 153 (19h,01h)    min =    0;  max =  255

E4_VOICE_CORD8_DST,        id = 154 (1Ah,01h)    min =    0;  max =  255

E4_VOICE_CORD8_AMT,        id = 155 (1Bh,01h)    min = -100;  max = +100

E4_VOICE_CORD9_SRC,        id = 156 (1Ch,01h)    min =    0;  max =  255

E4_VOICE_CORD9_DST,        id = 157 (1Dh,01h)    min =    0;  max =  255

E4_VOICE_CORD9_AMT,        id = 158 (1Eh,01h)    min = -100;  max = +100

E4_VOICE_CORD10_SRC,       id = 159 (1Fh,01h)    min =    0;  max =  255

E4_VOICE_CORD10_DST,       id = 160 (20h,01h)    min =    0;  max =  255

E4_VOICE_CORD10_AMT,       id = 161 (21h,01h)    min = -100;  max = +100

E4_VOICE_CORD11_SRC,       id = 162 (22h,01h)    min =    0;  max =  255

E4_VOICE_CORD11_DST,       id = 163 (23h,01h)    min =    0;  max =  255

E4_VOICE_CORD11_AMT,       id = 164 (24h,01h)    min = -100;  max = +100

E4_VOICE_CORD12_SRC,       id = 165 (25h,01h)    min =    0;  max =  255

E4_VOICE_CORD12_DST,       id = 166 (26h,01h)    min =    0;  max =  255

E4_VOICE_CORD12_AMT,       id = 167 (27h,01h)    min = -100;  max = +100

E4_VOICE_CORD13_SRC,       id = 168 (28h,01h)    min =    0;  max =  255

E4_VOICE_CORD13_DST,       id = 169 (29h,01h)    min =    0;  max =  255

E4_VOICE_CORD13_AMT,       id = 170 (2Ah,01h)    min = -100;  max = +100

E4_VOICE_CORD14_SRC,       id = 171 (2Bh,01h)    min =    0;  max =  255

E4_VOICE_CORD14_DST,       id = 172 (2Ch,01h)    min =    0;  max =  255

E4_VOICE_CORD14_AMT,       id = 173 (2Dh,01h)    min = -100;  max = +100

E4_VOICE_CORD15_SRC,       id = 174 (2Eh,01h)    min =    0;  max =  255

E4_VOICE_CORD15_DST,       id = 175 (2Fh,01h)    min =    0;  max =  255

E4_VOICE_CORD15_AMT,       id = 176 (30h,01h)    min = -100;  max = +100

E4_VOICE_CORD16_SRC,       id = 177 (31h,01h)    min =    0;  max =  255

E4_VOICE_CORD16_DST,       id = 178 (32h,01h)    min =    0;  max =  255

E4_VOICE_CORD16_AMT,       id = 179 (33h,01h)    min = -100;  max = +100

E4_VOICE_CORD17_SRC,       id = 180 (34h,01h)    min =    0;  max =  255

E4_VOICE_CORD17_DST,       id = 181 (35h,01h)    min =    0;  max =  255

E4_VOICE_CORD17_AMT,       id = 182 (36h,01h)    min = -100;  max = +100

There are a maximum of 256 Source and Destination settings(including Off).

They are grouped, and not numbered in a linear fashion.  Additions

may be included in the future.

Source

0  =  Off

4  =  XfdRnd   (Crossfade Random)

8  =  Key+     (Key 0...127)

9  =  Key~     (Key -64...+63)

10 =  Vel+     (Velocity 0...127)

11 =  Vel~     (Velocity -64...+63)

12 =  Vel<     (Velocity -127...0)

13 =  RlsVel   (Release Velocity)

14 =  Gate

16 =  PitWl    (Pitch Wheel)

17 =  ModWl    (Mod Wheel)

18 =  Press    (Pressure)

19 =  Pedal    (Pedal)

20 =  MidiA

21 =  MidiB

22 =  FtSw1    (Foot Switch 1)

23 =  FtSw2    (Foot Switch 2)

24 =  Ft1FF    (Flip-Flop Foot Switch 1)

25 =  Ft2FF    (Flip-Flop Foot Switch 2)

26 =  MidiVl   (Midi Volume controller 7)

27 =  MidPn    (Midi Pan controller 10)

32 =  MidiC

33 =  MidiD

34 =  MidiE

35 =  MidiF

36 =  MidiG

37 =  MidiH

38 =  Thumb

39 =  ThmFF

48 =  KeyGld   (Key Glide)

72 =  VEnv+    (Volume Envelope 0...127)

73 =  VEnv~    (Volume Envelope -64...+63)

74 =  VEnv<    (Volume Envelope -127...0)

80 =  FEnv+    (Filter Envelope 0...127)

81 =  FEnv~    (Filter Envelope -64...+63)

82 =  FEnv<    (Filter Envelope -127...0)

88 =  AEnv+    (Aux Envelope 0...127)

89 =  AEnv~    (Aux Envelope -64...+63)

90 =  AEnv<    (Aux Envelope -127...0)

96  =  Lfo1~

97  =  Lfo1+

98  =  White    (White Noise)

99  =  Pink     (Pink Noise)

100 =  kRand1   (kRandom 1)

101 =  kRand2   (kRandom 2)

104 =  Lfo2~

105 =  Lfo2+

106 =  Lag0in   (summing amp out)

107 =  Lag0

108 =  Lag1in   (summing amp out)

109 =  Lag1

144 =  CkDwhl   (Clock Double Whole Note)

145 =  CkWhle   (Clock Whole Note)

146 =  CkHalf   (Clock Half Note)

147 =  CkQtr    (Clock Quarter Note)

148 =  Ck8th    (Clock Eighth Note)

149 =  Ck16th   (Clock Sixteenth Note)

160 =  DC       (DC Offset)

161 =  Sum      (Summing Amp)

162 =  Switch

163 =  Abs      (Absolute Value)

164 =  Diode

165 =  FlipFlop

166 =  Quantiz  (Quantizer)

167 =  Gain4X

Destination

0 =   Off

8 =   KeySust

47 =  FinePtch

48 =  Pitch

49 =  Glide

50 =  ChrsAmt  (Chorus Amount)

51 =  'ChrsITD (Chorus Position ITD)

52 =  'SStart  (Sample Start)

53 =  SLoop    (Sample Loop)

54 =  SRetrig  (Sample Retrigger)

56 =  FilFreq  (Filter Frequency)

57 =  'FilRes  (Filter Resonance)

64 =  AmpVol   (Amplifier Volume)

65 =  AmpPan   (Amplifier Pan)

66 =  AmpXfd   (Amplifier Crossfade)

72 =  VEnvRts  (Volume Envelope Rates)

73 =  VEnvAtk  (Volume Envelope Attack)

74 =  VEnvDcy  (Volume Envelope Decay)

75 =  VEnvRls  (Volume Envelope Release)

80 =  FEnvRts  (Filter Envelope Rates)

81 =  FEnvAtk  (Filter Envelope Attack)

82 =  FEnvDcy  (Filter Envelope Decay)

83 =  FEnvRls  (Filter Envelope Release)

86 =  FEnvTrig (Filter Envelope Trigger/Retrigger)

88 =  AEnvRts  (Aux Envelope Rates)

89 =  AEnvAtk  (Aux Envelope Attack)

90 =  AEnvDcy  (Aux Envelope Decay)

91 =  AEnvRls  (Aux Envelope Release)

94 =  AEnvTrig (Aux Envelope Trigger/Retrigger)

96 =  Lfo1Rt   (Lfo 1 Rate)

97 =  Lfo1Trig (Lfo 1 Trigger/Retrigger)

104 =  Lfo2Rt   (Lfo 2 Rate)

105 =  Lfo2Trig (Lfo 2 Trigger/Retrigger)

106 =  Lag0in

108 =  Lag1in

161 =  Sum      (Summing Amp)

162 =  Switch

163 =  Abs      (Absolute Value)

164 =  Diode

165 =  FlipFlop

166 =  Quantize

167 =  Gain4X

168 =  C00Amt   (Cord Amount)

169 =  C01Amt

170 =  C02Amt

171 =  C03Amt

172 =  C04Amt

173 =  C05Amt

174 =  C06Amt

175 =  C07Amt

176 =  C08Amt

177 =  C09Amt

178 =  C10Amt

179 =  C11Amt

180 =  C12Amt

181 =  C13Amt

182 =  C14Amt

183 =  C15Amt

184 =  C16Amt

185 =  C17Amt

Master Mode Global Parms

Master Setup Tuning:

MASTER_TUNING_OFFSET,         id = 183 (37h,01h)    min = -64;  max = +64

display = +/-0.0

1.2

3.5

4.7

6.0

7.2

9.5

10.7

12.0

14.2

15.5

17.7

18.0

20.2

21.5

23.7

25.0

26.2

28.5

29.7

31.0

32.2

34.5

35.7

37.0

39.2

40.5

42.7

43.0

45.2

46.5

48.7

50.0

51.2

53.5

54.7

56.0

57.2

59.5

60.7

62.0

64.2

65.5

67.7

68.0

70.2

71.5

73.7

75.0

76.2

78.5

79.7

81.0

82.2

84.5

85.7

87.0

89.2

90.5

92.7

93.0

95.2

96.5

98.7

100.0

MASTER_TRANSPOSE,             id = 184 (38h,01h)    min = -12;  max = +12

display:

C  =   -12

C# =   -11

D  =   -10

D# =   -9

E  =   -8

F  =   -7

F# =   -6

G  =   -5

G# =   -4

A  =   -3

A# =   -2

B  =   -1

off (C) =    0

C# =   +1

D  =   +2

D# =   +3

E  =   +4

F  =   +5

F# =   +6

G  =   +7

G# =   +8

A  =   +9

A# =   +10

B  =   +11

C  =   +12

Master Setup Output:

MASTER_HEADROOM,              id = 185 (39h,01h)    min = 0;  max = 15

MASTER_HCHIP_BOOST,           id = 186 (3Ah,01h)    min = 0;  max = 1

Output Boost:  0 = +0dB

1 = +12dB

MASTER_OUTPUT_FORMAT,         id = 187 (3Bh,01h)    min = 0;  max = 2

Output Format:  0 = analog

1 = AES pro

2 = S/PDIF

MASTER_OUTPUT_CLOCK,          id = 188 (3Ch,01h)    min = 0;  max = 1

Output Clock:  0 = 44.1kHz

1 = 48kHz

MASTER_AES_BOOST,             id = 189 (3Dh,01h)    min = 0;  max = 1

AES Boost: 0 = off

1 = on

Master Setup Misc:

MASTER_SCSI_ID,               id = 190 (3Eh,01h)    min = 0;  max = 7

(SCSI ID)

MASTER_SCSI_TERM,             id = 191 (3Fh,01h)    min = 0;  max = 1

SCSI termination: 0 = on

1 = off

MASTER_USING_MAC,             id = 192 (40h,01h)    min = -1;  max = 7

Avoid Host on SCSI ID: -1 = none

0 = ID 0

1 = ID 1

2 = ID 2

3 = ID 3

4 = ID 4

5 = ID 5

6 = ID 6

7 = ID 7(Mac)

Master Setup Import:

MASTER_COMBINE_LR,            id = 193 (41h,01h)    min = 0;  max = 1

Combine L/R into stereo:  0 = on

1 = off

MASTER_AKAI_LOOP_ADJ,         id = 194 (42h,01h)    min = 0;  max = 1

Adjust Akai fractional loops:  0 = off

1 = on

MASTER_AKAI_SAMPLER_ID,       id = 195 (43h,01h)    min = -1;  max = 7

Foreign sampler SCSI ID:    -1 = none

0-7 = 0-7

***id's 196,197 not used.***

Master MIDI Mode:

MIDIGLO_BASIC_CHANNEL,        id = 198 (46h,01h)    min = 0;  max = 15(31 if has MIDI expansion card)

Basic Channel: 0-15(31) = 1-16(32)

MIDIGLO_MIDI_MODE,            id = 199 (47h,01h)    min = 0;  max = 2

MIDI mode: 0 = omni

1 = poly

2 = multi

***id 200 not used.***

Master MIDI Cntrls1:

MIDIGLO_PITCH_CONTROL,        id = 201 (49h,01h)    min = -1;  max = 33

MIDIGLO_MOD_CONTROL,          id = 202 (4Ah,01h)    min = -1;  max = 33

MIDIGLO_PRESSURE_CONTROL,     id = 203 (4Bh,01h)    min = -1;  max = 33

MIDIGLO_PEDAL_CONTROL,        id = 204 (4Ch,01h)    min = -1;  max = 33

Display for these 4 parameters:   -1 = off

0-31 = 0-31

32 = ptwheel

33 = chnpres

MIDIGLO_SWITCH_1_CONTROL,     id = 205 (4Dh,01h)    min = -1;  max = 33

MIDIGLO_SWITCH_2_CONTROL,     id = 206 (4Eh,01h)    min = -1;  max = 33

MIDIGLO_THUMB_CONTROL,        id = 207 (4Fh,01h)    min = -1;  max = 33

Display for these 3 parameters:    -1 = off

0-33 = 64-97

Master MIDI cntrls2:

MIDIGLO_MIDI_A_CONTROL,       id = 208 (50h,01h)    min = -1;  max = 33

MIDIGLO_MIDI_B_CONTROL,       id = 209 (51h,01h)    min = -1;  max = 33

MIDIGLO_MIDI_C_CONTROL,       id = 210 (52h,01h)    min = -1;  max = 33

MIDIGLO_MIDI_D_CONTROL,       id = 211 (53h,01h)    min = -1;  max = 33

MIDIGLO_MIDI_E_CONTROL,       id = 212 (54h,01h)    min = -1;  max = 33

MIDIGLO_MIDI_F_CONTROL,       id = 213 (55h,01h)    min = -1;  max = 33

MIDIGLO_MIDI_G_CONTROL,       id = 214 (56h,01h)    min = -1;  max = 33

MIDIGLO_MIDI_H_CONTROL,       id = 215 (57h,01h)    min = -1;  max = 33

Display for these 8 parameters:   -1 = off

0-31 = 0-31

32 = ptwheel

33 = chnpres

Master MIDI Prefs1:

MIDIGLO_VEL_CURVE,            id = 216 (58h,01h)    min = 0;  max = 13

Velocity Curve:   0 = linear

1-13 = 1-13

MIDIGLO_VOLUME_SENSITIVITY,   id = 217 (59h,01h)    min = 0;  max = 31

MIDIGLO_CTRL7_CURVE,          id = 218 (5Ah,01h)    min = 0;  max = 2

Ctrl#7 Curve:  0 = linear

1 = squared

2 = logarithmic

MIDIGLO_PEDAL_OVERRIDE,       id = 219 (5Bh,01h)    min = 0;  max = 1

Global pedal override:  0 = off

1 = on

Master MIDI Prefs2:

MIDIGLO_RCV_PROGRAM_CHANGE,   id = 220 (5Ch,01h)    min = 0;  max = 1

Rcv Program Change:  0 = off

1 = on

MIDIGLO_SEND_PROGRAM_CHANGE,  id = 221 (5Dh,01h)    min = 0;  max = 1

Send Program Change:  0 = off

1 = on

MIDIGLO_MAGIC_PRESET,         id = 222 (5Eh,01h)    min = 0;  max = 128

Magic load preset:      0 = off

1-128 = Presets 000 - 127

PRESET_SELECT,             id = 223 (5Fh,01h)    min = 000;  max = 999

LINK_SELECT,               id = 224 (60h,01h)    min =   0;  max = (255 - Num of Voices)

VOICE_SELECT,              id = 225 (61h,01h)    min =   0;  max = (255 - Num of links)

SAMPLE_ZONE_SELECT,        id = 226 (62h,01h)    min =   0;  max = 255

GROUP_SELECT,              id = 227 (63h,01h)    min =   0;  max = 31

These Selection Parameters are independant of what is selected from the front

pannel of the E4.  Once you select something remotely, the only way to change

the selection is remotely, with a couple of exceptions.  Any regular front pannel

editing can be done without affecting what was selected for remote editing.

A Preset Selection will always remain until you change it.

A Link Selection will remain until you select another Link, or if you select

another Preset, in which case no Links are selected.

Similar with a Voice Selection.

A Sample Zone selection will get reset if you select a new Voice or a new Preset.

Master Effects Parameter Section(similar to Preset Effects Parameters):

MASTER_FX_A_ALGORITHM,     id = 228 (64h,01h)   min = 0;  max =  44;  default = 14;

MASTER_FX_A_PARM_0,        id = 229 (65h,01h)   min = 0;  max =  90;  default = 54;

MASTER_FX_A_PARM_1,        id = 230 (66h,01h)   min = 0;  max = 127;  default = 64;

MASTER_FX_A_PARM_2,        id = 231 (67h,01h)   min = 0;  max = 127;  default = 0;

MASTER_FX_A_AMT_0,         id = 232 (68h,01h)   min = 0;  max = 100;  default = 10;

MASTER_FX_A_AMT_1,         id = 233 (69h,01h)   min = 0;  max = 100;  default = 20;

MASTER_FX_A_AMT_2,         id = 234 (6Ah,01h)   min = 0;  max = 100;  default = 30;

MASTER_FX_A_AMT_3,         id = 235 (6Bh,01h)   min = 0;  max = 100;  default = 40;

MASTER_FX_B_ALGORITHM,     id = 236 (6Ch,01h)   min = 0;  max =  27;  default = 1;

MASTER_FX_B_PARM_0,        id = 237 (6Dh,01h)   min = 0;  max = 127;  default = 0;

MASTER_FX_B_PARM_1,        id = 238 (6Eh,01h)   min = 0;  max = 127;  default = 3;

MASTER_FX_B_PARM_2,        id = 239 (6Fh,01h)   min = 0;  max = 127;  default = 0;

MASTER_FX_B_AMT_0,         id = 240 (70h,01h)   min = 0;  max = 100;  default = 10;

MASTER_FX_B_AMT_1,         id = 241 (71h,01h)   min = 0;  max = 100;  default = 15;

MASTER_FX_B_AMT_2,         id = 242 (72h,01h)   min = 0;  max = 100;  default = 30;

MASTER_FX_B_AMT_3,         id = 243 (73h,01h)   min = 0;  max = 100;  default = 0;

MASTER_FX_BYPASS,          id = 244 (74h,01h)   min =  0;  max =  1;  default =  0;

MASTER_FX_MM_CTRL_CHANNEL, id = 245 (75h,01h)   min = -1;  max = 15;  default = -1;

MULTIMODE_CHANNEL,         id = 246 (76h,01h)   min =   1;  max = 16(32);     default =   1;

MULTIMODE_PRESET,          id = 247 (77h,01h)   min =  -1;  max = 999(1255);  default =  -1;

MULTIMODE_VOLUME,          id = 248 (78h,01h)   min =   0;  max =  127;       default = 127;

MULTIMODE_PAN   ,          id = 249 (79h,01h)   min = -64;  max =  +63;       default =   0;

MULTIMODE_SUBMIX,          id = 250 (7Ah,01h)   min =  -1;  max = 3(7);       default =  -1;

Midi Expansion Card yields 32 total Channels.

Output Expansion Card yields Submix 4 - 7.

E4_LINK_INTERNAL_EXTERNAL, id = 251 (7Bh,01h)   min = 0;  max =  16;  default = 0;

E4_LINK_FILTER_PITCH,      id = 252 (7Ch,01h)   min = 0;  max =   1;  default = 0;

E4_LINK_FILTER_MOD,        id = 253 (7Dh,01h)   min = 0;  max =   1;  default = 0;

E4_LINK_FILTER_PRESSURE,   id = 254 (7Eh,01h)   min = 0;  max =   1;  default = 0;

E4_LINK_FILTER_PEDAL,      id = 255 (7Fh,01h)   min = 0;  max =   1;  default = 0;

E4_LINK_FILTER_CTRL_A,     id = 256 (00h,02h)   min = 0;  max =   1;  default = 0;

E4_LINK_FILTER_CTRL_B,     id = 257 (01h,02h)   min = 0;  max =   1;  default = 0;

E4_LINK_FILTER_CTRL_C,     id = 258 (02h,02h)   min = 0;  max =   1;  default = 0;

E4_LINK_FILTER_CTRL_D,     id = 259 (03h,02h)   min = 0;  max =   1;  default = 0;

E4_LINK_FILTER_CTRL_E,     id = 260 (04h,02h)   min = 0;  max =   1;  default = 0;

E4_LINK_FILTER_CTRL_F,     id = 261 (05h,02h)   min = 0;  max =   1;  default = 0;

E4_LINK_FILTER_CTRL_G,     id = 262 (06h,02h)   min = 0;  max =   1;  default = 0;

E4_LINK_FILTER_CTRL_H,     id = 263 (07h,02h)   min = 0;  max =   1;  default = 0;

E4_LINK_FILTER_SWITCH_1,   id = 264 (08h,02h)   min = 0;  max =   1;  default = 0;

E4_LINK_FILTER_SWITCH_2,   id = 265 (09h,02h)   min = 0;  max =   1;  default = 0;

E4_LINK_FILTER_THUMB,      id = 266 (0Ah,02h)   min = 0;  max =   1;  default = 0;

0 = filter off

1 = filter on

/** ULTRA ONLY PARAMETERS *************************************************/

MASTER_WORD_CLOCK_IN,          id = 267 (0Bh,02h)    min = 0;  max = 4

Word Clock In:  0 = Internal

1 = BNC

2 = AES

3 = ADAT

4 = (future expansion)

MASTER_WORD_CLOCK_PHASE_IN,    id = 268 (0Ch,02h)    min = 0;  max = 511

MASTER_WORD_CLOCK_PHASE_OUT,   id = 269 (0Dh,02h)    min = 0;  max = 511

0.00 - 359.30 degrees in 512 increments.

MASTER_OUTPUT_DITHER,          id = 270 (0Eh,02h)    min = 0;  max = 1

0 = off

1 = on

/** END ULTRA ONLY PARAMETERS ********************************************/

MASTER_AUDITION_KEY,           id = 271 (0Fh,02h)    min = 0;  max = 127

Effects Algorithms:

Preset Effect A:

(44 algorithms)

Master Effect A  (Global Effects Edit)

Room 1-3

Hall 1-2

Plate

Delay

Panning Delay

Multitap 1

Multitap Pan

3 Tap

3 Tap Pan

Soft Room

Warm Room

Perfect Room

Tiled Room

Hard Plate

Warm Hall

Spacious Hall

Bright Hall

Brt Hall Pan

Bright Plate

BBall Court

Gymnasium

Cavern

Concert 9

Concert 10 Pan

Reverse Gate

Gate 2

Gate Pan

Concert 11

MediumConcert

Large Concert

Lg Concert Pan

Canyon

DelayVerb 1-3

DelayVerb4Pan

DelayVerb5Pan

DelayVerb 6-9

Parameters:

E4_PRESET_FX_A_PARM_0 -> Decay Time: 0 - 90

E4_PRESET_FX_A_PARM_1 -> HF Damping: 0 - 127

E4_PRESET_FX_A_PARM_2 -> FxB==>FxA : 0 - 127

E4_PRESET_FX_A_AMT_0 ->  Main FX A Send: 0 - 100%

E4_PRESET_FX_A_AMT_1 ->  Sub1 FX A Send: 0 - 100%

E4_PRESET_FX_A_AMT_2 ->  Sub2 FX A Send: 0 - 100%

E4_PRESET_FX_A_AMT_3 ->  Sub3 FX A Send: 0 - 100%

Preset Effect B:

(27 algorithms)

Master Effect B (Global Effects Edit)

Chorus 1-5

Doubling

Slapback

Flange 1-7

Big Chorus

Symphonic

Ensemble

Delay

Delay Stereo

Delay Stereo 2

Panning Delay

Delay Chorus

Pan Dly Chrs 1-2

DualTap 1/3

DualTap 1/4

Vibrato

Distortion 1-2

DistortedFlange

DistortedChorus

DistortedDouble

Parameters:

E4_PRESET_FX_B_PARM_0 -> Feedback: 0 - 127

E4_PRESET_FX_B_PARM_1 -> LFO Rate: 0 - 127

E4_PRESET_FX_B_PARM_2 -> Delay Time: 0 - 127 (0 - 635ms in 5ms increments)

E4_PRESET_FX_B_AMT_0 ->  Main FX A Send: 0 - 100%

E4_PRESET_FX_B_AMT_1 ->  Sub1 FX A Send: 0 - 100%

E4_PRESET_FX_B_AMT_2 ->  Sub2 FX A Send: 0 - 100%

E4_PRESET_FX_B_AMT_3 ->  Sub3 FX A Send: 0 - 100%

For many filter parameters, I will need to supply tables to convert to

displayed values(Hz,dB).  I will include this information in a future document.

Filter Types:

2 Pole Low-pass:

4 Pole Low-pass:

6 Pole Low-pass:

E4_VOICE_FMORPH,           id = 83      min = 0;  max = 255

Frequency: 57Hz to 20000Hz (see Filter Table 1)

E4_VOICE_FKEY_XFORM,       id = 84      min = 0;  max = 127

Q: 0 to 127

2nd Order High-pass:

4th Order High-pass:

E4_VOICE_FMORPH,           id = 83      min = 0;  max = 255

Frequency: 69Hz to 18000Hz (see Filter Table 2)

E4_VOICE_FKEY_XFORM,       id = 84      min = 0;  max = 127

Q: 0 to 127

2nd Order Band-pass:

4th Order Band-pass:

Contrary Band-pass:

E4_VOICE_FMORPH,           id = 83      min = 0;  max = 255

Frequency: 57Hz to 10000Hz (see Filter Table 3)

E4_VOICE_FKEY_XFORM,       id = 84      min = 0;  max = 127

Q: 0 to 127

Swept EQ 1 octave:

Swept EQ 2->1 oct:

Swept EQ 3->1 oct:

E4_VOICE_FMORPH,           id = 83      min = 0;  max = 255

Frequency: 83Hz to 10000Hz (see Filter Table 3)

E4_VOICE_FKEY_XFORM,       id = 84      min = 0;  max = 127

Gain: -24.0 dB  to  +23.6 dB (see Filter Table 4)

Phaser 1:

Phaser 2:

Bat-Phaser:

Flanger Lite:

E4_VOICE_FMORPH,           id = 83      min = 0;  max = 255

Frequency: 0 to 255

E4_VOICE_FKEY_XFORM,       id = 84      min = 0;  max = 127

Resonance: 0 to 127

Vocal Ah-Ay-Ee:

Vocal Oo-Ah:

E4_VOICE_FMORPH,           id = 83      min = 0;  max = 255

Morph: 0 to 255

E4_VOICE_FKEY_XFORM,       id = 84      min = 0;  max = 127

Body Size: 0 to 127

Dual EQ Morph:

E4_VOICE_FMORPH,           id = 83      min = 0;  max = 255

Morph: 0 to 255

E4_VOICE_FKEY_XFORM,       id = 84      min = 0;  max = 127

Gain: -24.0dB to +23.6dB (see Filter Table 4)

E4_VOICE_FILT_GEN_PARM3,   id = 87      min = 0;  max = 127

EQ 1 Low: 83Hz to 9824Hz (see Filter Table 5)

E4_VOICE_FILT_GEN_PARM4,   id = 88      min = 0;  max = 127

EQ 1 High: 83Hz to 9824Hz (see Filter Table 5)

E4_VOICE_FILT_GEN_PARM5,   id = 89      min = 0;  max = 127

EQ 1 Gain: -24.0dB to +23.6dB (see Filter Table 4)

E4_VOICE_FILT_GEN_PARM6,   id = 90      min = 0;  max = 127

EQ 2 Low: 83Hz to 9824Hz (see Filter Table 5)

E4_VOICE_FILT_GEN_PARM7,   id = 91      min = 0;  max = 127

EQ 2 High: 83Hz to 9824Hz (see Filter Table 5)

E4_VOICE_FILT_GEN_PARM8,   id = 92      min = 0;  max = 127

EQ 2 Gain: -24.0dB to +23.6dB (see Filter Table 4)

2EQ+Lowpass Morph:

E4_VOICE_FMORPH,           id = 83      min = 0;  max = 255

Fc/Morph: 0 to 255

E4_VOICE_FKEY_XFORM,       id = 84      min = 0;  max = 127

LPF Q: 0 to 127

E4_VOICE_FILT_GEN_PARM3,   id = 87      min = 0;  max = 127

EQ 1 Low: 83Hz to 9824Hz (see Filter Table 5)

E4_VOICE_FILT_GEN_PARM4,   id = 88      min = 0;  max = 127

EQ 1 High: 83Hz to 9824Hz (see Filter Table 5)

E4_VOICE_FILT_GEN_PARM5,   id = 89      min = 0;  max = 127

EQ 1 Gain: -24.0dB to +23.6dB (see Filter Table 4)

E4_VOICE_FILT_GEN_PARM6,   id = 90      min = 0;  max = 127

EQ 2 Low: 83Hz to 9824Hz (see Filter Table 5)

E4_VOICE_FILT_GEN_PARM7,   id = 91      min = 0;  max = 127

EQ 2 High: 83Hz to 9824Hz (see Filter Table 5)

E4_VOICE_FILT_GEN_PARM8,   id = 92      min = 0;  max = 127

EQ 2 Gain: -24.0dB to +23.6dB (see Filter Table 4)

2EQMorph+Exprssn:

E4_VOICE_FMORPH,           id = 83      min = 0;  max = 255

Morph: 0 to 255

E4_VOICE_FKEY_XFORM,       id = 84      min = 0;  max = 127

Expression: 0 to 127

E4_VOICE_FILT_GEN_PARM3,   id = 87      min = 0;  max = 127

EQ 1 Low: 83Hz to 9824Hz (see Filter Table 5)

E4_VOICE_FILT_GEN_PARM4,   id = 88      min = 0;  max = 127

EQ 1 High: 83Hz to 9824Hz (see Filter Table 5)

E4_VOICE_FILT_GEN_PARM5,   id = 89      min = 0;  max = 127

EQ 1 Gain: -24.0dB to +23.6dB (see Filter Table 4)

E4_VOICE_FILT_GEN_PARM6,   id = 90      min = 0;  max = 127

EQ 2 Low: 83Hz to 9824Hz (see Filter Table 5)

E4_VOICE_FILT_GEN_PARM7,   id = 91      min = 0;  max = 127

EQ 2 High: 83Hz to 9824Hz (see Filter Table 5)

E4_VOICE_FILT_GEN_PARM8,   id = 92      min = 0;  max = 127

EQ 2 Gain: -24.0dB to +23.6dB (see Filter Table 4)

Peak/Shelf Morph:

E4_VOICE_FMORPH,           id = 83      min = 0;  max = 255

Morph: 0 to 255

E4_VOICE_FKEY_XFORM,       id = 84      min = 0;  max = 127

Peak: -24.0dB to +23.6dB (see Filter Table 4)

E4_VOICE_FILT_GEN_PARM3,   id = 87      min = 0;  max = 127

low morph frame Freq: 83Hz to 9824Hz (see Filter Table 5)

E4_VOICE_FILT_GEN_PARM4,   id = 88      min = 0;  max = 127

low morph frame Shelf: -64 to 63

E4_VOICE_FILT_GEN_PARM5,   id = 89      min = 0;  max = 127

low morph frame Peak: -24.0dB to +23.6dB (see Filter Table 4)

E4_VOICE_FILT_GEN_PARM6,   id = 90      min = 0;  max = 127

high morph frame Freq: 83Hz to 9824Hz (see Filter Table 5)

E4_VOICE_FILT_GEN_PARM7,   id = 91      min = 0;  max = 127

high morph frame Shelf: -64 to 63

E4_VOICE_FILT_GEN_PARM8,   id = 92      min = 0;  max = 127

high morph frame Peak: -24.0dB to +23.6dB (see Filter Table 4)

FILTER Hz and dB DISPLAY TABLE CALCULATIONS:

/*

*

* example fil_freq (100, 20000, 1002)

* in=0..255

*/

int fil_freq (int input, int maxfreq, int mul)

{

int f = maxfreq;

input = 255 - input;

while (input-- > 0)

f *= mul, f /= 1024;

return f;

}

/*in=0..255*/

void cnv_morph_freq (int input, char *buf)

{

sprintf (buf, "%dHz", fil_freq (input, 10000, 1006)); /*1009*/

}

/* in=0..127 out=-24..+24    (32in ==> 12out) */

void cnv_morph_gain (int input, char *buf)

{

int gain10x = -240 + ((input * 120) / 32);

int gain_i  = gain10x / 10;

int gain_f  = abs (gain10x % 10);

sprintf (buf, "%s%d.%1ddB",

gain10x >= 0 ? "+" : "-",

abs (gain_i),

gain_f);

}

Filter Table 1:    sprintf (value, "%dHz", fil_freq (input, 20000, 1002)); /* input=0..255 */

Filter Table 2:    sprintf (value, "%dHz", fil_freq (input, 18000, 1003)); /* input=0..255 */

Filter Table 3:    sprintf (value, "%dHz", fil_freq (input, 10000, 1006)); /*1009; input=0..255 */

Filter Table 4:    cnv_morph_gain (input, value); /* input=0..127 */

Filter Table 5:    cnv_morph_freq (2*input, value); /* input=0..127 */

/*****************************************************************************************************/

/*****************************************************************************************************/

Commands:

00h          Reserved.  If received, the following 2 MIDI bytes form a

14 bit command.

01h          Parameter Edit

02h          Parameter Value Request

03h          Parameter min/max/default Request

04h          Parameter min/max/default Response

05h          Preset Name

06h          Preset Name Request

07h          Preset Name Character Update

08h          Preset Name Character Request

09h          Sample Name

0Ah          Sample Name Request

0Bh          Sample Name Character Update

0Ch          Sample Name Character Request

0Dh          Preset Dump/NEW Preset Dump/NEW Preset Dump Request

0Eh          Preset Dump Request

Configuration Utils:

10h          Preset Memory Request

11h          Preset Memory Response

12h          Sample Memory Request

13h          Sample Memory Response

14h          Configuration Request (Hardware options installed, amount of RAM etc.)

15h          Configuration Response

16h          Preset Number of Voices Request

17h          Preset Number of Voices Response

18h          Preset Number of Links Request

19h          Preset Number of Links Response

1Ah          Preset Number of SZones Request

1Bh          Preset Number of SZones Response

1Ch          Voice Number of SZones Request

1Dh          Voice Number of SZones Response

1Eh          Extended Configuration Request (Hardware options installed, amount of RAM etc.)

1Fh          Extended Configuration Response

Voice Utils:

20h          New Voice

21h          Delete Voice

22h          Copy Voice

SZone Utils:

30h          New Sample Zone

31h          Get Multisample

32h          Delete Sample Zone

33h          Combine

34h          Expand

Link Utils:

40h          New Link

41h          Delete Link

42h          Copy Link

Sample Utils:

50h          Sample Erase

52h          Sample Memory Defrag

Misc Utils:

70h          Copy Preset

71h          Delete Preset

72h          Multimode Map Dump

73h          Multimode Map Dump Request

74h          Erase RAM Bank

75h          Erase All RAM Presets

76h          Erase All RAM Samples

Dump/Handshaking:

79h          NEW Dump Nak

7Ah          NEW Dump Ack

7Bh          EOF

7Ch          Wait

7Dh          Cancel

7Eh          Nak

7Fh          Ack

Parameter Value Edit:

EXAMPLE:> {F0h,18h,21h,ddh,55h,01h,02h,<xxh,xxh,yyh,yyh>,<Checksum>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

01h = Command::Parameter Value Edit

02h = Byte count(number of Byte pairs)

{xxh,xxh} = Parameter ID (LSB first)

{yyh,yyh} = Parameter Data

<Checksum> = 1 Byte = XOR(1's compliment) of sum of {<xxh,xxh,yyh,yyh>}

= 7Fh is ignore checksum flag.

F7h = EOX

This message contains only 1 packet.

Do not send messages containing more than 256 Data Bytes, or 42 Parameter Edits, at a time.

Parameter Value Request:

EXAMPLE:> {F0h,18h,21h,ddh,55h,02h,01h,xxh,xxh,<Checksum>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

02h = Command::Parameter Value Request

01h = Byte count(number of Byte pairs)

{xxh,xxh} = Parameter ID (LSB first)

<Checksum> = 1 Byte = XOR(1's compliment) of sum of {xxh,xxh}

F7h = EOX

This message contains only 1 request packet.

The response a complete Parameter Value Edit SYSEX message for each parameter.

Do not send messages containing more than 256 Data Bytes, or 64 Parameter IDs, at a time.

Parameter Min/Max/Default Value Request:

This is a Read Only system.

EXAMPLE:> {F0h,18h,21h,ddh,55h,03h,xxh,xxh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

03h = Command::Parameter Min/Max/Default Value Request

xxh,xxh = The Parameter ID

F7h = EOX

RESPONSE:> {F0h,18h,21h,ddh,55h,04h,xxh,xxh,yyh,yyh,zzh,zzh,qqh,qqh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

04h = Command::Parameter Min/Max/Default

xxh,xxh = the Parameter ID

yyh,yyh = Parameter minimum value

zzh,zzh = Parameter maximum value

qqh,qqh = Parameter default value

F7h = EOX

Preset Name:

EXAMPLE:> {F0h,18h,21h,ddh,55h,05h,xxh,xxh,<STRING 1>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

05h = Command::ASCII Preset Name

xxh,xxh = Preset number 0-999

<STRING x> = <CHAR 1>,<CHAR 2>,...,<CHAR 16> = Preset Name

F7h = EOX

Preset Name Request:

EXAMPLE:> {F0h,18h,21h,ddh,55h,06h,xxh,xxh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

06h = Command::ASCII Preset Name Request

xxh,xxh = Preset number 0-999

F7h = EOX

The response is the ASCII Preset Name SYSEX message.

Preset Name Character Update:

EXAMPLE:> {F0h,18h,21h,ddh,55h,07h,xxh,xxh,yyh,zzh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

07h = Command::Preset Name Character Update

<xxh,xxh> = Preset number, LSB first

yyh = Character number (0-15)

zzh = ASCII character

F7h = EOX

Preset Name Character Request:

EXAMPLE:> {F0h,18h,21h,ddh,55h,08h,xxh,xxh,yyh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

08h = Command::Preset Name Character Request

<xxh,xxh> = Preset number, LSB first

yyh = Character number (0-15)

F7h = EOX

response is the Preset Name Character Update message.

Sample Name:

EXAMPLE:> {F0h,18h,21h,ddh,55h,09h,xxh,xxh,<STRING 1>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

09h = Command::ASCII Sample Name

xxh,xxh = Sample number 000-999

<STRING x> = <CHAR 1>,<CHAR 2>,...,<CHAR 16> = Sample Name

F7h = EOX

Sample Name Request:

EXAMPLE:> {F0h,18h,21h,ddh,55h,0Ah,xxh,xxh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

0Ah = Command::ASCII Sample Name Request

xxh,xxh = Sample number 000-999(2999)

F7h = EOX

The response is the ASCII Sample Name SYSEX message.

Sample Name Character Update:

EXAMPLE:> {F0h,18h,21h,ddh,55h,0Bh,xxh,xxh,yyh,zzh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

0Bh = Command::Sample Name Character Update

<xxh,xxh> = Sample number, LSB first

yyh = Character number (0-15)

zzh = ASCII character

F7h = EOX

Sample Name Character Request:

EXAMPLE:> {F0h,18h,21h,ddh,55h,0Ch,xxh,xxh,yyh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

0Ch = Command::Sample Name Character Request

<xxh,xxh> = Sample number, LSB first

yyh = Character number (0-15)

F7h = EOX

response is the Sample Name Character Update message.

Dump Format:

The transfer of large messages shall follow a method similar to the

MIDI Sample Dump Standard.

Generic Handshaking Messages:

ACK:

EXAMPLE:> {F0h,18h,21h,ddh,55h,7Fh,pph,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

7Fh = Command::Acknowledge; The last packet was received correctly.

pph = Packet number

F7h = EOX

NAK:

EXAMPLE:> {F0h,18h,21h,ddh,55h,7Eh,pph,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

7Eh = Command::Negative Acknowledge; The last packet was received

incorrectly, resend.

pph = Packet number

F7h = EOX

CANCEL:

EXAMPLE:> {F0h,18h,21h,ddh,55h,7Dh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

7Dh = Command::Cancel; Abort the Dump.

F7h = EOX

WAIT:

EXAMPLE:> {F0h,18h,21h,ddh,55h,7Ch,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

7Ch = Command::Wait; Stop sending packets until an ACK is received.

F7h = EOX

EOF:

EXAMPLE:> {F0h,18h,21h,ddh,55h,7Bh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

7Bh = Command::End Of File; No more packets follow, no response required.

Must be sent at end of transfer.

F7h = EOX

When a Dump is requested or initiated, the first element of the Dump

sent is the Dump Header.  This contains information on what kind of

Dump it is, and how many Bytes are included in the Data.  What follows

are 266 Byte messages that each contain 256 Bytes of actual Data.  The

last message contains 256 Bytes, or LESS, depending on how much Data is left.

The total number of these messages, and the number of Bytes (256 or less)

of Data contained in the final message can be derived from the total

number of Bytes in this Header.  Generic handshaking messages

will be used to negotiate the transfer.  After all the Data messages

have been sent, and End Of File will be sent, closing the transfer.

Examples of the Dump Header, and the different types of data within

the Data messages.

DUMP HEADER:

EXAMPLE:> {F0h,18h,21h,ddh,55h,0Dh,01h,00h,kkh,kkh,kkh,kkh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

0Dh = Command::Preset Dump;

01h = subCommand::Dump Header;

00h = Packet Number (first packet)

kkh,kkh,kkh,kkh = Number of bytes, LSB first.

F7h = EOX

DATA MESSAGE:

EXAMPLE:> {F0h,18h,21h,ddh,55h,0Dh,02h,kkh,<256 Data Bytes>,<Checksum>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

0Dh = Command::Preset Dump

02h = subCommand::Dump Message;

kkh = Running Packet count(low 7 bits in long dumps)

<256 Data Bytes> (except for last message)

<Checksum> = 1 Byte = 1's compliment of the sum of {<256 Data Bytes>}

F7h = EOX

Dump Data Formats:

/* short form */

PresetDump        :== PresetNumber Name GlobalParms NoOfLinks { Link }*NoOfLinks

NoOfVoices { Voice }*NoOfVoices.

PresetNumber      :== 0 .. 999.

Name              :== 16 * ASCIIChar.

GlobalParms       :== E4_PRESET_TRANSPOSE ... E4_PRESET_FX_B_AMT_3.

NoOfLinks         :== MAX(0 - 255, 255-NoOfVoices).

Link              :== E4_LINK_PRESET ... E4_LINK_VEL_HIGHFADE.

NoOfVoices        :== MAX(1 - 255, 255-NoOfLinks).

Voice             :== E4_GEN_GROUP_NUM SingleSampleVoice | MultiSampleVoice.

SingleSampleVoice :== E4_GEN_SAMPLE VoiceParams 1.

MultiSampleVoice  :== 3FFFh VoiceParams NoOfZones { Zone }*NoOfZones.

VoiceParams       :== E4_GEN_VOLUME ... E4_VOICE_CORD17_AMT.

NoOfZones         :== 0 .. 255.

Zone              :== E4_GEN_SAMPLE E4_GEN_VOLUME E4_GEN_PAN E4_GEN_FTUNE E4_GEN_ORIG_KEY

E4_GEN_KEY_LOW ... E4_GEN_VEL_HIGHFADE.

/* long form */

Preset Dump:

<256 Data Bytes> x <? messages> = {<NUMBER>,<NAME>,<DATA 1>,<DATA 2>,...,}

(last message may contain less than 256 Data Bytes)

<NUMBER> = Preset number

<NAME> = Preset Name, 16 ASCII characters

<DATA 1>,<DATA 2>,..., = (see below)

The Data is organized in the following way, and then broken into 256 Byte chunks.

{<NUMBER>,<NAME>,<Global Parms>,<Links>,<Voices>}

<NUMBER>:

Preset number (000 - 999)

<NAME>:

16 ASCII characters.

<Global Parms>:

Always starts with the first 6 parms.  All parms are 2 Bytes.

E4_PRESET_TRANSPOSE,       id = 0       min = -24;  max = +24  (semitones)

E4_PRESET_VOLUME,          id = 1       min = -96;  max = +10  (dB)

E4_PRESET_CTRL_A,          id = 2       min = -1;   max = 127  (-1 = off)

E4_PRESET_CTRL_B,          id = 3       min = -1;   max = 127  (-1 = off)

E4_PRESET_CTRL_C,          id = 4       min = -1;   max = 127  (-1 = off)

E4_PRESET_CTRL_D,          id = 5       min = -1;   max = 127  (-1 = off)

Then effects A and effects B.  If the effects A or B Algorithm is 0, then the

effects parameters are the values of Master Effects A or B.  After the effects,

the next word is the beginning of the LINKS section of Data.

<Links>:

The first word is the number of Links in the Preset.

The max number of Links + the max number of Voices = 256.

Each Link has 13 parameters, so the number of Bytes remaining

in this section is (13 words * 2 Bytes/word * number of Links).

These 13 word blocks are delivered in order of Link number,

starting with 1.  If there are no Links in the Preset then

no Link data follows the Link count word.

<Voices>:

The first word is the number of Voices in the Preset.

The max number of Voices + the max number of Links = 256.

66 Bytes of Preset so far if no Links.

The next group of words applies to the first Voice.

The first word is the Group number associated with

the Voice.

What follows are the Voice Parameters.  If the Sample

Number is 3FFFh, then it is a "multisample" voice.

The first word following the Voice Parameters is the number

of Sample Zones.  If the number of Sample Zones is 1, the

Voice is not a "multisample", and the next word is the

beginning of the next Voice(the Group number).  If the number

of Sample Zones is greater than 1, then Sample Zone Parameters

corresponding to the number of Sample Zones follows, and

after the last Sample Zone of the Voice, the next set of Voice

parameters follow, assuming another Voice exists.  If

no more Voices exist after the last Sample Zone parameter

of the current voice, then the Preset is complete.

The Voice Parameters consist of all the General Parameters(20),

Tuning(11), Amp/Filt(37), Lfo/Aux(24), and Cords(54) groups

of parameters.  There are 146 total base(single Sample Voice)

parameters per Voice.  Additional Samples require additional

parameters.  This number along with the number of Samples word

= 147 words, or 294 Bytes.

The Sample Parameters consist of a subset of the General

Parameters.  Each additional Sample requires a block of

information containing the Sample number, and the 12 Sample

parameters.  There can be 255 of these additional Sample zones

per Voice.

E4_GEN_SAMPLE,             id = 38      min =   0;  max = 999(2999)

E4_GEN_VOLUME,             id = 39      min = -96;  max = +10

E4_GEN_PAN,                id = 40      min = -64;  max = +63

E4_GEN_FTUNE,              id = 42      min = -64;  max = +64

E4_GEN_ORIG_KEY,           id = 44      min =   0;  max = 127  (60 = C3)

E4_GEN_KEY_LOW,            id = 45      min = 0;  max = 127  (C-2 -> G8)

E4_GEN_KEY_LOWFADE,        id = 46      min = 0;  max = 127

E4_GEN_KEY_HIGH,           id = 47      min = 0;  max = 127  (C-2 -> G8)

E4_GEN_KEY_HIGHFADE,       id = 48      min = 0;  max = 127

E4_GEN_VEL_LOW,            id = 49      min = 0;  max = 127

E4_GEN_VEL_LOWFADE,        id = 50      min = 0;  max = 127

E4_GEN_VEL_HIGH,           id = 51      min = 0;  max = 127

E4_GEN_VEL_HIGHFADE,       id = 52      min = 0;  max = 127

The total Max number of words per Voice becomes 146 + (255*13) = 3461.

Each parameter is 2 bytes, for a total of 6922 Bytes.  Each preset can

have a maximum of 256 Voices(assuming no Links).  The first word is

the number of Voices, and only occurs once, so the total Bytes in a

maximum sized Preset = 6920 * 256 = 1,771,520 Bytes.  This is just to

give you an idea of the Theoretical limits, however it is advisable

to keep the Presets to 64k or less.

Preset Dump Request:

EXAMPLE:> {F0h,18h,21h,ddh,55h,0Eh,xxh,xxh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

0Eh = Command::Preset Parameter Dump Request

<xxh,xxh> = Preset number

F7h = EOX

The response is the Preset Dump SYSEX message.

*If a non-existant Preset is requested, the response is a CANCEL message.

*WARNING!  Only 1 Preset may be Dumped to or from the E4 at a time!

=====================================================================================

NEW and IMPROVED Dump Format:

The transfer of large messages shall follow a method similar to the

MIDI Sample Dump Standard.

Preset Dumps of the Old format may still be Requested from and Dumped to the E4.

Generic Handshaking Messages:

NEW ACK:

EXAMPLE:> {F0h,18h,21h,ddh,55h,7Ah,<pph,pph>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

7Ah = Command::Acknowledge; The last packet was received correctly.

<pph,pph> = Packet number

F7h = EOX

NEW NAK:

EXAMPLE:> {F0h,18h,21h,ddh,55h,79h,<pph,pph>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

79h = Command::Negative Acknowledge; The last packet was received

incorrectly, resend.

<pph,pph> = Packet number

F7h = EOX

CANCEL:

EXAMPLE:> {F0h,18h,21h,ddh,55h,7Dh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

7Dh = Command::Cancel; Abort the Dump.

F7h = EOX

WAIT:

EXAMPLE:> {F0h,18h,21h,ddh,55h,7Ch,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

7Ch = Command::Wait; Stop sending packets until an ACK is received.

F7h = EOX

EOF:

EXAMPLE:> {F0h,18h,21h,ddh,55h,7Bh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

7Bh = Command::End Of File; No more packets follow, no response required.

Must be sent at end of transfer.

F7h = EOX

When a Dump is requested or initiated, the first element of the Dump

sent is the Dump Header.  This contains information on the type of Dump

(old or new), The Preset Number of the Dump, how many Bytes are included

in the Data, and the number of Parameters in each section, which may

depend on the version of EOS the Dump may have come from.  As parameters

are added to newer version of EOS, the dump format can expand with that.

The Dump Engine within older EOS versions is smart enough to ignore

Parameters it does not understand if a newer dump is transmitted to that

old EOS Software.

What follows the Header message are 255-Byte messages that each contain

244 Bytes of actual Data.  The last message may contain LESS than 255

Bytes, depending on how much Data is left.  Generic handshaking messages

will be used to negotiate the transfer.  After all the Data messages

have been sent, an End Of File message will be sent, closing the transfer.

Examples of the NEW Dump Header, and the different types of data within

the NEW Data messages, follow.

NEW DUMP HEADER:

EXAMPLE:> {F0h,18h,21h,ddh,55h,0Dh,03h,nnh,nnh,kkh,kkh,kkh,kkh,ggh,ggh,llh,llh,vvh,vvh,zzh,zzh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

0Dh = Command::Preset Dump;

03h = subCommand::NEW Dump Header;

<nnh,nnh> = Preset Number.

<kkh,kkh,kkh,kkh> = Number of Data bytes in the Dump, LSB first.

<ggh,ggh> = Number of Global Parameters, LSB first.

<llh,llh> = Number of Link Parameters per Link, LSB first.

<vvh,vvh> = Number of Voice Parameters per Voice, LSB first.

<zzh,zzh> = Number of Sample Zone Parameters per Sample Zone, LSB first.

F7h = EOX

NEW DATA MESSAGE:

EXAMPLE:> {F0h,18h,21h,ddh,55h,0Dh,04h,kkh,kkh,<244 Data Bytes>,<Checksum>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

0Dh = Command::Preset Dump

04h = subCommand::NEW Dump Message;

<kkh,kkh> = Running Packet count, LSB first, begins at 1

<244 Data Bytes> (except for last message)

<Checksum> = 1 Byte = 1's compliment of the sum of {<244 Data Bytes>}

F7h = EOX

NEW Dump Data Formats:

/* short form */

PresetDump        :== Name GlobalParms NoOfLinks { Link }*NoOfLinks

NoOfVoices { Voice }*NoOfVoices.

Name              :== 16 * ASCIIChar.

GlobalParms       :== E4_PRESET_TRANSPOSE ... (end of Global Parameters).

NoOfLinks         :== MAX(0 - 255, 255-NoOfVoices).

Link              :== E4_LINK_PRESET ... (end of Link Parameters).

NoOfVoices        :== MAX(1 - 255, 255-NoOfLinks).

Voice             :== E4_GEN_GROUP_NUM SingleSampleVoice | MultiSampleVoice.

SingleSampleVoice :== E4_GEN_SAMPLE VoiceParams 1.

MultiSampleVoice  :== 3FFFh VoiceParams NoOfZones { Zone }*NoOfZones.

VoiceParams       :== E4_GEN_VOLUME ... (end of Voice Parameters).

NoOfZones         :== 0 .. 255.

Zone              :== E4_GEN_SAMPLE E4_GEN_VOLUME E4_GEN_PAN E4_GEN_FTUNE E4_GEN_ORIG_KEY

E4_GEN_KEY_LOW ... (end of Sample Zone Parameters).

/* long form */

Preset Dump:

<244 Data Bytes> x <? messages> = {<NAME>,<DATA 1>,<DATA 2>,...,}

(last message may contain less than 244 Data Bytes)

<NAME> = Preset Name, 16 ASCII characters

<DATA 1>,<DATA 2>,..., = (see below)

The Data is organized in the following way, and then broken into 244 Byte chunks.

Preset Name:           16 Bytes

Global Parms:          44 Bytes

Number of Links:        2 Bytes

Link Parms:       (58 Bytes * Number of Links)

Number of Voices:       2 Bytes

Voice Parms:     (292 Bytes * Number of Voices)

Number of SZones   (2 Bytes * Number of Voices)

SZone Parms   (26 Bytes * Number of SZones per Voice)

1 Voice, no Links, no SZones: 358 Data Bytes

{<NAME>,<Global Parms>,<Links>,<Voices>}

<NAME>:

16 ASCII characters, 16 Bytes.

<Global Parms>:

There are currently 22 Global Parameters, for a total of 44 Bytes.

Always starts with the first 6 parms.  All parms are 2 Bytes.

E4_PRESET_TRANSPOSE,       id = 0       min = -24;  max = +24  (semitones)

E4_PRESET_VOLUME,          id = 1       min = -96;  max = +10  (dB)

E4_PRESET_CTRL_A,          id = 2       min = -1;   max = 127  (-1 = off)

E4_PRESET_CTRL_B,          id = 3       min = -1;   max = 127  (-1 = off)

E4_PRESET_CTRL_C,          id = 4       min = -1;   max = 127  (-1 = off)

E4_PRESET_CTRL_D,          id = 5       min = -1;   max = 127  (-1 = off)

Then effects A and effects B.  If the effects A or B Algorithm is 0, then the

effects parameters are the values of Master Effects A or B.  After the effects,

the next word is the beginning of the LINKS section of Data.  More Global

Parameters may be added to future versions of EOS, in which case they would

follow these Global Effects parameters, but before the LINK parameters.  The

number of Global Parameters is contained in the Header message.

<Links>:

There are currently 29 LINK parameters, or 58 Bytes per LINK.

The first word(2 Bytes, LSB first) is the number of Links in the Preset.

The max number of Links + the max number of Voices = 256.

Each Link currently has 29 parameters, so the number of Bytes

remaining in this section is (29 words * 2 Bytes/word * number of Links).

These 28 word blocks are delivered in order of Link number,

starting with 1.  If there are no Links in the Preset then

no Link data follows the 0 Link count word.

<Voices>:

62 Bytes of Preset so far if no Links.

The first word is the number of Voices in the Preset.

The max number of Voices + the max number of Links = 256.

The next group of words applies to the first Voice.

The first word is the Group number associated with

the Voice.

What follows are the Voice Parameters.

The first word following the Voice Parameters is the number

of Sample Zones.

- If the Sample Number is 3FFFh, then it is a "multisample" voice.

- If the number of Sample Zones is 1, the Voice is not a "multisample",

and the next word is the beginning of the next Voice(the Group number).

- If the number of Sample Zones is greater than 1, then Sample Zone

Parameters corresponding to the number of Sample Zones follows, and

after the last Sample Zone of the Voice, the next set of Voice

parameters follow, assuming another Voice exists.

If no more Voices exist after the last Sample Zone parameter

of the current voice, then the Preset is complete.

The Voice Parameters currently consist of:

- General Parameters(20 if single Sample Zone, 19 if a MultiSample Voice),

- Tuning(11)

- Amp/Filt(37)

- Lfo/Aux(24)

- Cords(54)

There are 146 total base(single Sample Voice) parameters per Voice.

This number along with the Number Of Samples word = 147 words, or 294 Bytes

for a Single Sample Zone Voice.

Additional Sample Zones require an additional 13 parameters per Zone, but

1 less parameter in the Voice(The Sample Original Key Parameter).

The Sample Parameters consist of a subset of the General

Parameters.  Each additional Sample requires a block of

information containing the Sample number, and the 13 current Sample

parameters.  There can be 255 of these additional Sample zones

per Voice.

E4_GEN_SAMPLE,             id = 38      min =   0;  max = 999(2999)

E4_GEN_VOLUME,             id = 39      min = -96;  max = +10

E4_GEN_PAN,                id = 40      min = -64;  max = +63

E4_GEN_FTUNE,              id = 42      min = -64;  max = +64

E4_GEN_ORIG_KEY,           id = 44      min =   0;  max = 127  (60 = C3)

E4_GEN_KEY_LOW,            id = 45      min = 0;  max = 127  (C-2 -> G8)

E4_GEN_KEY_LOWFADE,        id = 46      min = 0;  max = 127

E4_GEN_KEY_HIGH,           id = 47      min = 0;  max = 127  (C-2 -> G8)

E4_GEN_KEY_HIGHFADE,       id = 48      min = 0;  max = 127

E4_GEN_VEL_LOW,            id = 49      min = 0;  max = 127

E4_GEN_VEL_LOWFADE,        id = 50      min = 0;  max = 127

E4_GEN_VEL_HIGH,           id = 51      min = 0;  max = 127

E4_GEN_VEL_HIGHFADE,       id = 52      min = 0;  max = 127

The total Max number of words per Voice becomes 147 + (255*13) = 3462.

Each parameter is 2 bytes, for a total of 6924 Bytes.  Each preset can

have a maximum of 256 Voices(assuming no Links).  So 256*6924 = 1772544 Bytes.

Add 62 Bytes for the Name, Globals, Number of Links, and Number of

Voices and the total comes to 1,772,606 Bytes!  This is just to

give you an idea of the Theoretical limits, however it is advisable

to keep the Presets to 64k or less.

Preset Dump Request:

EXAMPLE:> {F0h,18h,21h,ddh,55h,0Dh,05h,xxh,xxh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

0Dh = Command::Preset Dump

05h = subCommand::NEW Preset Dump Request;

<xxh,xxh> = Preset number

F7h = EOX

The response is the Preset Dump SYSEX message.

*If a non-existant Preset is requested, the response is a CANCEL message.

*WARNING!  Only 1 Preset may be Dumped to or from the E4 at a time!

====================================================================================

Configuration Utilities:

Preset Memory Request:

This is a Read Only system.

EXAMPLE:> {F0h,18h,21h,ddh,55h,10h,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

10h = Command::Preset Memory Request

F7h = EOX

RESPONSE:> {F0h,18h,21h,ddh,55h,11h,xxh,xxh,yyh,yyh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

11h = Command::Preset Memory Response

<xxh,xxh> = Total Preset Memory (in kBytes) LSB first

<yyh,yyh> = Total Preset Memory Free (in kBytes) LSB first

F7h = EOX

Sample Memory Request:

EXAMPLE:> {F0h,18h,21h,ddh,55h,12h,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

12h = Command::Sample Memory Request

F7h = EOX

RESPONSE:> {F0h,18h,21h,ddh,55h,13h,xxh,xxh,yyh,yyh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

13h = Command::Sample Memory Response

<xxh,xxh> = Total Sample Memory (in MBytes) LSB first

<yyh,yyh> = Total Sample Memory Free (in 10's of kBytes) LSB first

F7h = EOX

Configuration Request:

This is a Read Only system.

EXAMPLE:> {F0h,18h,21h,ddh,55h,14h,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

14h = Command::Configuration Request

F7h = EOX

RESPONSE:> {F0h,18h,21h,ddh,55h,15h,<Options>,<RAM>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

15h = Command::Configuration Response

<Options> = 7 bit number, each bit representing if an option is

present or not.

bit 0: 0 = 64 Voices; 1 = 128 Voice

bit 1: 0 = No FX Card; 1 = FX Card installed

bit 2: 0 = No MIDI card; 1 = MIDI card installed

bit 3: 0 = No Octopus card; 1 = Octopus card installed

bit 4: 0 = No Digital I/O; 1 = Digital I/O installed

bit 5: 0 = <undefined>

bit 6: 0 = <undefined>

<RAM>   = 2 Byte Sample RAM configuration(0 - 128 MB)

F7h = EOX

Preset Number Of Voices Request:

This is a Read Only system.

EXAMPLE:> {F0h,18h,21h,ddh,55h,16h,<xxh,xxh>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

16h = Command::Preset Num Of Voices Request

<xxh,xxh> = Preset Number (LSB first)

F7h = EOX

RESPONSE:> {F0h,18h,21h,ddh,55h,17h,<Num_Of_Voices>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

17h = Command::Preset Num Of Voices Response

<Num_Of_Voices> = 2 Byte number of Voices.

F7h = EOX

Preset Number Of Links Request:

This is a Read Only system.

EXAMPLE:> {F0h,18h,21h,ddh,55h,18h,<xxh,xxh>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

18h = Command::Preset Num Of Links Request

<xxh,xxh> = Preset Number (LSB first)

F7h = EOX

RESPONSE:> {F0h,18h,21h,ddh,55h,19h,<Num_Of_Links>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

19h = Command::Preset Num Of Links Response

<Num_Of_Voices> = 2 Byte number of Links.

F7h = EOX

Preset Number Of Sample Zones Request:

This is a Read Only system.

EXAMPLE:> {F0h,18h,21h,ddh,55h,1Ah,<xxh,xxh>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

1Ah = Command::Preset Num Of SZones Request

<xxh,xxh> = Preset Number (LSB first)

F7h = EOX

RESPONSE:> {F0h,18h,21h,ddh,55h,1Bh,<Num_Of_SZones>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

1Bh = Command::Preset Num Of SZones Response

<Num_Of_SZones> = 2 Byte number of SZones.

F7h = EOX

Voice Number Of Sample Zones Request:

This is a Read Only system.

EXAMPLE:> {F0h,18h,21h,ddh,55h,1Ch,<xxh,xxh>,<yyh,yyh>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

1Ch = Command::Voice Num Of SZones Request

<xxh,xxh> = Preset Number (LSB first)

<yyh,yyh> = Voice Number (LSB first)

F7h = EOX

RESPONSE:> {F0h,18h,21h,ddh,55h,1Dh,<V_Num_Of_SZones>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

1Dh = Command::Voice Num Of SZones Response

<V_Num_Of_SZones> = 2 Byte number of SZones.

F7h = EOX

Expanded Configuration Request:

This is a Read Only system.

EXAMPLE:> {F0h,18h,21h,ddh,55h,1Eh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

1Eh = Command::Configuration Request

F7h = EOX

RESPONSE:> {F0h,18h,21h,ddh,55h,1Fh,<Options1>,<Options2>,<RAM>,<ROM>,<Flash>,<reserved>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

1Fh = Command::Configuration Response

<Options1> = 7 bit number, each bit representing if an option is

present or not.

bit 0: 0 = 64 Voices; 1 = 128 Voice

bit 1: 0 = No FX Card; 1 = FX Card installed

bit 2: 0 = No MIDI card; 1 = MIDI card installed

bit 3: 0 = No Octopus card; 1 = Octopus card installed

bit 4: 0 = No Digital I/O; 1 = Digital I/O installed

bit 5: 0 = No Preset Flash; 1 = Preset Flash upgrade installed

bit 6: 0 = No ADAT I/O; 1 = ADAT I/O installed

<Options2> = 7 bit number, each bit representing if an option is

present or not.  Reserved for future expansion.

bit 0: 0 = <undefined>

bit 1: 0 = <undefined>

bit 2: 0 = <undefined>

bit 3: 0 = <undefined>

bit 4: 0 = <undefined>

bit 5: 0 = <undefined>

bit 6: 0 = <undefined>

<RAM>   = 2 Byte Sample RAM configuration(0 - 128 MB)

<ROM>   = 1 Byte Sample ROM configuration(0 - 128 MB) (0x7f = 128)

<Flash> = 1 Byte Sample Flash configuration(0 - 128 MB) (0x7f = 128)

<reserved> = 4 Bytes reserved for future expansion.

F7h = EOX

Voice Utility Commands:

New Voice:

Adds a new Voice to the Current Preset.

EXAMPLE:> {F0h,18h,21h,ddh,55h,20h,xxh,xxh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

20h = Command::New Voice

xxh,xxh = Preset Number

F7h = EOX

Delete Voice:

Deletes a Voice from the Current Preset.

EXAMPLE:> {F0h,18h,21h,ddh,55h,21h,xxh,xxh,yyh,yyh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

21h = Command::Delete Voice

xxh,xxh = Preset Number

yyh,yyh = Voice number (0 -> 255)

F7h = EOX

Copy Voice:

Copy a Voice to a new Voice in the Current Preset.

EXAMPLE:> {F0h,18h,21h,ddh,55h,22h,xxh,xxh,yyh,yyh,zzh,zzh,ggh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

22h = Command::Copy Voice

xxh,xxh = Source Preset (000 -> 999) LSB first

yyh,yyh = Source Voice  (0 -> 255) LSB first

zzh,zzh = Destination Preset (000 -> 999) LSB first

ggh     = Group Number (0 -> 31)

F7h = EOX

Sample Zone Commands:

New Sample Zone:

Adds a new Sample Zone to the currently selected Voice in the Current Preset.

EXAMPLE:> {F0h,18h,21h,ddh,55h,30h,xxh,xxh,yyh,yyh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

30h = Command::New Sample Zone

xxh,xxh = Preset Number

yyh,yyh = Voice Number

F7h = EOX

Get Multisample:

Replace Samples in currently selected Voice with Samples from any other

Voice in the Bank.

EXAMPLE:> {F0h,18h,21h,ddh,55h,31h,xxh,xxh,yyh,yyh,zzh,zzh,qqh,qqh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

31h = Command::Get Multisample

xxh,xxh = Source Preset (000 -> 999)

yyh,yyh = Source Voicenum

zzh,zzh = Destination Preset

qqh,qqh = Destination Voicenum

F7h = EOX

Delete Sample Zone:

Delete the currently selected Sample Zone in the Current Preset.

EXAMPLE:> {F0h,18h,21h,ddh,55h,32h,xxh,xxh,yyh,yyh,zzh,zzh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

32h = Command::Delete Sample Zone

xxh,xxh = Preset Number

yyh,yyh = Voice Number

zzh,zzh = Sample Number

F7h = EOX

Combine:

Take all the Voices in the given group and Combine them into one Voice.

EXAMPLE:> {F0h,18h,21h,ddh,55h,33h,xxh,xxh,yyh,yyh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

33h = Command::Combine

xxh,xxh = Preset Number

yyh,yyh = Group number (0 -> 31)

F7h = EOX

Expand:

Expand all the Sample Zones in the currently selected Multisample Voice

into separate Voices for each Sample Zone.

EXAMPLE:> {F0h,18h,21h,ddh,55h,34h,xxh,xxh,yyh,yyh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

34h = Command::Expand

xxh,xxh = Preset Number

yyh,yyh = Voice Number

F7h = EOX

Link Utility Commands:

New Link:

Adds a new Link to the Current Preset.

EXAMPLE:> {F0h,18h,21h,ddh,55h,40h,xxh,xxh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

40h = Command::New Link

xxh,xxh = Preset Number

F7h = EOX

Delete Link:

Delete currently selected Link from the Current Preset.

EXAMPLE:> {F0h,18h,21h,ddh,55h,41h,xxh,xxh,yyh,yyh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

41h = Command::Delete Link

xxh,xxh = Preset Number

yyh,yyh = Link Number

F7h = EOX

Sample Defragment Memory:

EXAMPLE:> {F0h,18h,21h,ddh,55h,52h,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

52h = Command::Sample Defragment Memory

F7h = EOX

Copy Link:

Copy a Link from any Preset to any Preset.

EXAMPLE:> {F0h,18h,21h,ddh,55h,42h,xxh,xxh,yyh,yyh,zzh,zzh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

42h = Command::Copy Link

xxh,xxh = Source Preset (000 -> 999)

yyh,yyh = Source Link (0 -> 255)

zzh,zzh = Destination Preset (000 -> 999)

F7h = EOX

Sample Commands

Sample Erase:

EXAMPLE:> {F0h,18h,21h,ddh,55h,50h,xxh,xxh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

50h = Command::Sample Erase

xxh,xxh = Sample Number to Erase

F7h = EOX

Sample Defragment Memory:

EXAMPLE:> {F0h,18h,21h,ddh,55h,52h,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

52h = Command::Sample Defragment Memory

F7h = EOX

Preset Copy:

EXAMPLE:> {F0h,18h,21h,ddh,55h,70h,xxh,xxh,yyh,yyh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

70h = Command::Preset Copy

xxh,xxh = Source Preset number 0-999(1255)

yyh,yyh = Destination Preset number 0-999(1255)

F7h = EOX

Preset Copy destroys whatever Preset existed in the Destination location.

If There is not enough Preset memory to copy the Preset, the copy is

aborted and a NAK message is sent as a reply.

Preset Delete:

EXAMPLE:> {F0h,18h,21h,ddh,55h,71h,xxh,xxh,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

71h = Command::Preset Delete

xxh,xxh = Preset number 0-999(1255)

F7h = EOX

Multimode Map Dump:

EXAMPLE:> {F0h,18h,21h,ddh,55h,72h,<multimode map>,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

72h = Command::Multimode Map Dump

<multimode map> = 128 Bytes for 16 MIDI Channels

256 Bytes for 32 MIDI Channels (if expansion card)

F7h = EOX

The structure of a Multimode Map is like a string of Parameters without

their id's.  Each Channel has 4 parameters, or 8 Bytes.  The Channel number and Parameter

id's are assumed by the fixed order of the message.  For 16 channels this means 8 Bytes * 16,

or 128 Bytes.  For 32 Channels it is double, or 256 Bytes.  If the receiving

machine is sent a 32 Channel Multimode Map and does not have the Midi expansion

card, it will Truncate the message and only read in the first 16 channels.

If a Multimode Dump is requested from a machine with no Midi expansion card,

it will only generate a dump of the 16 Channels.

Ch.1 Preset  (2 Bytes)

Ch.1 Volume  (2 Bytes)

Ch.1 Pan     (2 Bytes)

Ch.1 Submix  (2 Bytes)

Ch.2 Preset  (2 Bytes)

Ch.2 Volume  (2 Bytes)

Ch.2 Pan     (2 Bytes)

Ch.2 Submix  (2 Bytes)

Ch.3 Preset  (2 Bytes)

Ch.3 Volume  (2 Bytes)

Ch.3 Pan     (2 Bytes)

Ch.3 Submix  (2 Bytes)

...

Multimode Map Dump Request:

EXAMPLE:> {F0h,18h,21h,ddh,55h,73h,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

73h = Command::Multimode Map Dump Request

F7h = EOX

Erase Current RAM Bank:

EXAMPLE:> {F0h,18h,21h,ddh,55h,74h,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

74h = Command::Erase Current RAM Bank

F7h = EOX

Erase All RAM Presets:

EXAMPLE:> {F0h,18h,21h,ddh,55h,75h,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

75h = Command::Erase All RAM Presets

F7h = EOX

Erase All RAM Samples:

EXAMPLE:> {F0h,18h,21h,ddh,55h,76h,F7h}

F0h = sysex message

18h = EMU ID

21h = E4 ID

ddh = Device ID

55h = Special Editor designator byte

76h = Command::Erase All RAM Samples

F7h = EOX